/*! \file

NetCDF Users Guide

\page user_guide The NetCDF Users Guide

\ref netcdf_summary

Introduction

- \ref netcdf_interface

- \ref netcdf_format

- \ref performance

- \ref archival

- \ref attribute_conventions

- \ref background

- \ref limitations

Components of a NetCDF Data Set

- \ref data_model

- \ref dimensions

- \ref variables

- \ref coordinate_variables

- \ref attributes

- \ref differences_atts_vars

Data

- \ref external_types

- \ref classic_structures

- \ref user_defined_types

- \ref type_conversion

- \ref data_access

- \ref remote_client

- \ref type_conversion

File Structure and Performance

- \ref classic_file_parts

- \ref parts_of_netcdf4

- \ref xdr_layer

- \ref large_file_support

- \ref offset_format_limitations

- \ref classic_format_limitations

- \ref netcdf_3_io

- \ref parallel_access

- \ref interoperability_with_hdf5

- \ref creating_self

- \ref dap_support

Improving Performance with Chunking

- \ref chunk_cache

- \ref default_chunking_4_1

- \ref default_chunking_4_0_1

- \ref chunking_parallel_io

- \ref bm_file

NetCDF Utilities

- \ref cdl_syntax

- \ref cdl_data_types

- \ref cdl_constants

- \ref guide_ncgen

- \ref guide_ncdump

- \ref guide_nccopy

- \ref guide_ncgen3

File Format Specification

- \ref classic_format_spec

- \ref computing_offsets

- \ref offset_examples

- \ref offset_format_spec

- \ref netcdf_4_spec

- \ref netcdf_4_classic_spec

- \ref hdf4_sd_format

\page netcdf_summary Summary

The purpose of the Network Common Data Form (netCDF) interface is to

allow you to create, access, and share array-oriented data in a form

that is self-describing and portable. "Self-describing" means that a

dataset includes information defining the data it contains. "Portable"

means that the data in a dataset is represented in a form that can be

accessed by computers with different ways of storing integers,

characters, and floating-point numbers. Using the netCDF interface for

creating new datasets makes the data portable. Using the netCDF

interface in software for data access, management, analysis, and

display can make the software more generally useful.

The netCDF software includes C, Fortran 77, Fortran 90, and C++

interfaces for accessing netCDF data. These libraries are available

for many common computing platforms.

The community of netCDF users has contributed ports of the software to

additional platforms and interfaces for other programming languages as

well. Source code for netCDF software libraries is freely available to

encourage the sharing of both array-oriented data and the software

that makes the data useful.

This User's Guide presents the netCDF data model. It explains how the

netCDF data model uses dimensions, variables, and attributes to store

data.

Reference documentation for UNIX systems, in the form of UNIX 'man'

pages for the C and FORTRAN interfaces is also available at the netCDF

web site (http://www.unidata.ucar.edu/netcdf), and with the netCDF

distribution.

The latest version of this document, and the language specific guides,

can be found at the netCDF web site,

http://www.unidata.ucar.edu/netcdf/docs, along with extensive

additional information about netCDF, including pointers to other

software that works with netCDF data.

Separate documentation of the Java netCDF library can be found at

http://www.unidata.ucar.edu/software/netcdf-java.

\page netcdf_interface The NetCDF Interface

The Network Common Data Form, or netCDF, is an interface to a library

of data access functions for storing and retrieving data in the form

of arrays. An array is an n-dimensional (where n is 0, 1, 2, ...)

rectangular structure containing items which all have the same data

type (e.g., 8-bit character, 32-bit integer). A scalar (simple single

value) is a 0-dimensional array.

NetCDF is an abstraction that supports a view of data as a collection

of self-describing, portable objects that can be accessed through a

simple interface. Array values may be accessed directly, without

knowing details of how the data are stored. Auxiliary information

about the data, such as what units are used, may be stored with the

data. Generic utilities and application programs can access netCDF

datasets and transform, combine, analyze, or display specified fields

of the data. The development of such applications has led to improved

accessibility of data and improved re-usability of software for

array-oriented data management, analysis, and display.

The netCDF software implements an abstract data type, which means that

all operations to access and manipulate data in a netCDF dataset must

use only the set of functions provided by the interface. The

representation of the data is hidden from applications that use the

interface, so that how the data are stored could be changed without

affecting existing programs. The physical representation of netCDF

data is designed to be independent of the computer on which the data

were written.

Unidata supports the netCDF interfaces for C (see <a

href="http://www.unidata.ucar.edu/netcdf/docs/netcdf-c.html#Top" >NetCDF C Interface

Guide</a>), FORTRAN 77 (see <a

href="http://www.unidata.ucar.edu/netcdf/docs/netcdf-f77.html#Top" >NetCDF Fortran 77

Interface Guide</a>), FORTRAN 90 (see <a

href="http://www.unidata.ucar.edu/netcdf/docs/netcdf-f90.html#Top" >NetCDF Fortran 90

Interface Guide</a>), and C++ (see <a

href="http://www.unidata.ucar.edu/netcdf/docs/netcdf-cxx.html#Top" >NetCDF C++ Interface

Guide</a>).

The netCDF library is supported for various UNIX operating systems. A

MS Windows port is also available. The software is also ported and

tested on a few other operating systems, with assistance from users

with access to these systems, before each major release. Unidata's

netCDF software is freely available <a

href="ftp://ftp.unidata.ucar.edu/pub/netcdf">via FTP</a> to encourage

its widespread use.

For detailed installation instructions, see <a

href="http://www.unidata.ucar.edu/netcdf/docs/building.html" >Building NetCDF</a>.

\page netcdf_format The netCDF File Format

Until version 3.6.0, all versions of netCDF employed only one binary

data format, now referred to as netCDF classic format. NetCDF classic

is the default format for all versions of netCDF.

In version 3.6.0 a new binary format was introduced, 64-bit offset

format. Nearly identical to netCDF classic format, it uses 64-bit

offsets (hence the name), and allows users to create far larger

datasets.

In version 4.0.0 a third binary format was introduced: the HDF5

format. Starting with this version, the netCDF library can use HDF5

files as its base format. (Only HDF5 files created with netCDF-4 can

be understood by netCDF-4).

By default, netCDF uses the classic format. To use the 64-bit offset

or netCDF-4/HDF5 format, set the appropriate constant when creating

the file.

To achieve network-transparency (machine-independence), netCDF classic

and 64-bit offset formats are implemented in terms of an external

representation much like XDR (eXternal Data Representation, see

http://www.ietf.org/rfc/rfc1832.txt), a standard for describing and

encoding data. This representation provides encoding of data into

machine-independent sequences of bits. It has been implemented on a

wide variety of computers, by assuming only that eight-bit bytes can

be encoded and decoded in a consistent way. The IEEE 754

floating-point standard is used for floating-point data

representation.

Descriptions of the overall structure of netCDF classic and 64-bit

offset files are provided later in this manual. See Structure.

The details of the classic and 64-bit offset formats are described in

an appendix. See File Format. However, users are discouraged from

using the format specification to develop independent low-level

software for reading and writing netCDF files, because this could lead

to compatibility problems if the format is ever modified.

\section select_format How to Select the Format

With three different base formats, care must be taken in creating data

files to choose the correct base format.

The format of a netCDF file is determined at create time.

When opening an existing netCDF file the netCDF library will

transparently detect its format and adjust accordingly. However,

netCDF library versions earlier than 3.6.0 cannot read 64-bit offset

format files, and library versions before 4.0 can't read netCDF-4/HDF5

files. NetCDF classic format files (even if created by version 3.6.0

or later) remain compatible with older versions of the netCDF library.

Users are encouraged to use netCDF classic format to distribute data,

for maximum portability.

To select 64-bit offset or netCDF-4 format files, C programmers should

use flag NC_64BIT_OFFSET or NC_NETCDF4 in function nc_create. See

nc_create.

In Fortran, use flag nf_64bit_offset or nf_format_netcdf4 in function

NF_CREATE. See NF_CREATE.

It is also possible to change the default creation format, to convert

a large body of code without changing every create call. C programmers

see nc_set_default_format. Fortran programs see NF_SET_DEFAULT_FORMAT.

\subsection classic_format NetCDF Classic Format

The original netCDF format is identified using four bytes in the file

header. All files in this format have “CDF\001” at the beginning of

the file. In this documentation this format is referred to as “netCDF

classic format.”

NetCDF classic format is identical to the format used by every

previous version of netCDF. It has maximum portability, and is still

the default netCDF format.

For some users, the various 2 GiB format limitations of the classic

format become a problem. (see Classic Limitations).  1.4.2 NetCDF

64-bit Offset Format

For these users, 64-bit offset format is a natural choice. It greatly

eases the size restrictions of netCDF classic files (see 64 bit Offset

Limitations).

Files with the 64-bit offsets are identified with a “CDF\002” at the

beginning of the file. In this documentation this format is called

“64-bit offset format.”

Since 64-bit offset format was introduced in version 3.6.0, earlier

versions of the netCDF library can't read 64-bit offset files.

\subsection netcdf_4_format NetCDF-4 Format

In version 4.0, netCDF included another new underlying format: HDF5.

NetCDF-4 format files offer new features such as groups, compound

types, variable length arrays, new unsigned integer types, parallel

I/O access, etc. None of these new features can be used with classic

or 64-bit offset files.

NetCDF-4 files can't be created at all, unless the netCDF configure

script is run with –enable-netcdf-4. This also requires version 1.8.0

of HDF5.

For the netCDF-4.0 release, netCDF-4 features are only available from

the C and Fortran interfaces. We plan to bring netCDF-4 features to

the CXX API in a future release of netCDF.

NetCDF-4 files can't be read by any version of the netCDF library

previous to 4.0. (But they can be read by HDF5, version 1.8.0 or

better).

For more discussion of format issues see The NetCDF Tutorial.

\page performance What about Performance?

One of the goals of netCDF is to support efficient access to small

subsets of large datasets. To support this goal, netCDF uses direct

access rather than sequential access. This can be much more efficient

when the order in which data is read is different from the order in

which it was written, or when it must be read in different orders for

different applications.

The amount of overhead for a portable external representation depends

on many factors, including the data type, the type of computer, the

granularity of data access, and how well the implementation has been

tuned to the computer on which it is run. This overhead is typically

small in comparison to the overall resources used by an

application. In any case, the overhead of the external representation

layer is usually a reasonable price to pay for portable data access.

Although efficiency of data access has been an important concern in

designing and implementing netCDF, it is still possible to use the

netCDF interface to access data in inefficient ways: for example, by

requesting a slice of data that requires a single value from each

record. Advice on how to use the interface efficiently is provided in

Structure.

The use of HDF5 as a data format adds significant overhead in metadata

operations, less so in data access operations. We continue to study

the challenge of implementing netCDF-4/HDF5 format without

compromising performance.

\page creating_self Creating Self-Describing Data conforming to Conventions

The mere use of netCDF is not sufficient to make data

"self-describing" and meaningful to both humans and machines. The

names of variables and dimensions should be meaningful and conform to

any relevant conventions. Dimensions should have corresponding

coordinate variables (See \ref coordinate_variables) where sensible.

Attributes play a vital role in providing ancillary information. It is

important to use all the relevant standard attributes using the

relevant conventions. For a description of reserved attributes (used

by the netCDF library) and attribute conventions for generic

application software, see Attribute Conventions.

A number of groups have defined their own additional conventions and

styles for netCDF data. Descriptions of these conventions, as well as

examples incorporating them can be accessed from the netCDF

Conventions site, http://www.unidata.ucar.edu/netcdf/conventions.html.

These conventions should be used where suitable. Additional

conventions are often needed for local use. These should be

contributed to the above netCDF conventions site if likely to interest

other users in similar areas.

\page limitations Limitations of NetCDF

The netCDF classic data model is widely applicable to data that can be

organized into a collection of named array variables with named

attributes, but there are some important limitations to the model and

its implementation in software. Some of these limitations have been

removed or relaxed in netCDF-4 files, but still apply to netCDF

classic and netCDF 64-bit offset files.

Currently, netCDF classic and 64-bit offset formats offer a limited

number of external numeric data types: 8-, 16-, 32-bit integers, or

32- or 64-bit floating-point numbers. (The netCDF-4 format adds 64-bit

integer types and unsigned integer types.)

With the netCDF-4/HDF5 format, new unsigned integers (of various

sizes), 64-bit integers, and the string type allow improved expression

of meaning in scientific data. The new VLEN (variable length) and

COMPOUND types allow users to organize data in new ways.

With the classic netCDF file format, there are constraints that limit

how a dataset is structured to store more than 2 GiBytes (a GiByte is

2^30 or 1,073,741,824 bytes, as compared to a Gbyte, which is

1,000,000,000 bytes.) of data in a single netCDF dataset. (see Classic

Limitations). This limitation is a result of 32-bit offsets used for

storing relative offsets within a classic netCDF format file. Since

one of the goals of netCDF is portable data, and some file systems

still can't deal with files larger than 2 GiB, it is best to keep

files that must be portable below this limit. Nevertheless, it is

possible to create and access netCDF files larger than 2 GiB on

platforms that provide support for such files (see Large File

Support).

The new 64-bit offset format allows large files, and makes it easy to

create to create fixed variables of about 4 GiB, and record variables

of about 4 GiB per record. (see 64 bit Offset Limitations). However,

old netCDF applications will not be able to read the 64-bit offset

files until they are upgraded to at least version 3.6.0 of netCDF

(i.e. the version in which 64-bit offset format was introduced).

With the netCDF-4/HDF5 format, size limitations are further relaxed,

and files can be as large as the underlying file system

supports. NetCDF-4/HDF5 files are unreadable to the netCDF library

before version 4.0.

Another limitation of the classic (and 64-bit offset) model is that

only one unlimited (changeable) dimension is permitted for each netCDF

data set. Multiple variables can share an unlimited dimension, but

then they must all grow together. Hence the classic netCDF model does

not permit variables with several unlimited dimensions or the use of

multiple unlimited dimensions in different variables within the same

dataset. Variables that have non-rectangular shapes (for example,

ragged arrays) cannot be represented conveniently.

In netCDF-4/HDF5 files, multiple unlimited dimensions are fully

supported. Any variable can be defined with any combination of limited

and unlimited dimensions.

The extent to which data can be completely self-describing is limited:

there is always some assumed context without which sharing and

archiving data would be impractical. NetCDF permits storing meaningful

names for variables, dimensions, and attributes; units of measure in a

form that can be used in computations; text strings for attribute

values that apply to an entire data set; and simple kinds of

coordinate system information. But for more complex kinds of metadata

(for example, the information necessary to provide accurate

georeferencing of data on unusual grids or from satellite images), it

is often necessary to develop conventions.

Specific additions to the netCDF data model might make some of these

conventions unnecessary or allow some forms of metadata to be

represented in a uniform and compact way. For example, adding explicit

georeferencing to the netCDF data model would simplify elaborate

georeferencing conventions at the cost of complicating the model. The

problem is finding an appropriate trade-off between the richness of

the model and its generality (i.e., its ability to encompass many

kinds of data). A data model tailored to capture the shared context

among researchers within one discipline may not be appropriate for

sharing or combining data from multiple disciplines.

The classic netCDF data model (which is used for classic-format and

64-bit offset format data) does not support nested data structures

such as trees, nested arrays, or other recursive structures. Through

use of indirection and conventions it is possible to represent some

kinds of nested structures, but the result may fall short of the

netCDF goal of self-describing data.

In netCDF-4/HDF5 format files, the introduction of the compound type

allows the creation of complex data types, involving any combination

of types. The VLEN type allows efficient storage of ragged arrays, and

the introduction of hierarchical groups allows users new ways to

organize data.

Finally, using the netCDF-3 programming interfaces, concurrent access

to a netCDF dataset is limited. One writer and multiple readers may

access data in a single dataset simultaneously, but there is no

support for multiple concurrent writers.

NetCDF-4 supports parallel read/write access to netCDF-4/HDF5 files,

using the underlying HDF5 library and parallel read/write access to

classic and 64-bit offset files using the parallel-netcdf library.

For more information about HDF5, see the HDF5 web site:

http://hdfgroup.org/HDF5/.

For more information about parallel-netcdf, see their web site:

http://www.mcs.anl.gov/parallel-netcdf.

\page data_model The Data Model

A netCDF dataset contains dimensions, variables, and attributes, which

all have both a name and an ID number by which they are

identified. These components can be used together to capture the

meaning of data and relations among data fields in an array-oriented

dataset. The netCDF library allows simultaneous access to multiple

netCDF datasets which are identified by dataset ID numbers, in

addition to ordinary file names.

\section Enhanced Data Model in NetCDF-4/HDF5 Files

Files created with the netCDF-4 format have access to an enhanced data

model, which includes named groups. Groups, like directories in a Unix

file system, are hierarchically organized, to arbitrary depth. They

can be used to organize large numbers of variables.

\image html nc4-model.png "Enhanced NetCDF Data Model"

\image latex nc4-model.png "Enhanced NetCDF Data Model"

\image rtf nc4-model.png "Enhanced NetCDF Data Model"

Each group acts as an entire netCDF dataset in the classic model. That

is, each group may have attributes, dimensions, and variables, as well

as other groups.

The default group is the root group, which allows the classic netCDF

data model to fit neatly into the new model.

Dimensions are scoped such that they can be seen in all descendant

groups. That is, dimensions can be shared between variables in

different groups, if they are defined in a parent group.

In netCDF-4 files, the user may also define a type. For example a

compound type may hold information from an array of C structures, or a

variable length type allows the user to read and write arrays of

variable length values.

Variables, groups, and types share a namespace. Within the same group,

variables, groups, and types must have unique names. (That is, a type

and variable may not have the same name within the same group, and

similarly for sub-groups of that group.)

Groups and user-defined types are only available in files created in

the netCDF-4/HDF5 format. They are not available for classic or 64-bit

offset format files.

\page object_name Name

\section Permitted Characters in NetCDF Names

The names of dimensions, variables and attributes (and, in netCDF-4

files, groups, user-defined types, compound member names, and

enumeration symbols) consist of arbitrary sequences of alphanumeric

characters, underscore '_', period '.', plus '+', hyphen '-', or at

sign '@', but beginning with an alphanumeric character or

underscore. However names commencing with underscore are reserved for

system use.

Beginning with versions 3.6.3 and 4.0, names may also include UTF-8

encoded Unicode characters as well as other special characters, except

for the character '/', which may not appear in a name.

Names that have trailing space characters are also not permitted.

Case is significant in netCDF names.

\section Name Length

A zero-length name is not allowed.

Names longer than ::NC_MAX_NAME will not be accepted any netCDF define

function. An error of ::NC_EMAXNAME will be returned.

All netCDF inquiry functions will return names of maximum size

::NC_MAX_NAME for netCDF files. Since this does not include the

terminating NULL, space should be reserved for NC_MAX_NAME + 1

characters.

\section Conventions

Some widely used conventions restrict names to only alphanumeric

characters or underscores.

\page archival Is NetCDF a Good Archive Format?

NetCDF classic or 64-bit offset formats can be used as a

general-purpose archive format for storing arrays. Compression of data

is possible with netCDF (e.g., using arrays of eight-bit or 16-bit

integers to encode low-resolution floating-point numbers instead of

arrays of 32-bit numbers), or the resulting data file may be

compressed before storage (but must be uncompressed before it is

read). Hence, using these netCDF formats may require more space than

special-purpose archive formats that exploit knowledge of particular

characteristics of specific datasets.

With netCDF-4/HDF5 format, the zlib library can provide compression on

a per-variable basis. That is, some variables may be compressed,

others not. In this case the compression and decompression of data

happen transparently to the user, and the data may be stored, read,

and written compressed.

\page attribute_conventions Attribute Conventions

Attribute conventions are assumed by some netCDF generic applications,

e.g., ‘units’ as the name for a string attribute that gives the units

for a netCDF variable.

It is strongly recommended that applicable conventions be followed

unless there are good reasons for not doing so. Below we list the

names and meanings of recommended standard attributes that have proven

useful. Note that some of these (e.g. units, valid_range,

scale_factor) assume numeric data and should not be used with

character data.

\note Attribute names commencing with underscore ('_') are reserved

for use by the netCDF library.

\section units

A character string that specifies the units used for the variable's

data. Unidata has developed a freely-available library of routines to

convert between character string and binary forms of unit

specifications and to perform various useful operations on the binary

forms. This library is used in some netCDF applications. Using the

recommended units syntax permits data represented in conformable units

to be automatically converted to common units for arithmetic

operations. For more information see Units.

\section long_name

A long descriptive name. This could be used for labeling plots, for

example. If a variable has no long_name attribute assigned, the

variable name should be used as a default.

\section _FillValue

The _FillValue attribute specifies the fill value used to pre-fill

disk space allocated to the variable. Such pre-fill occurs unless

nofill mode is set using nc_set_fill(). The fill value is returned

when reading values that were never written. If ::_FillValue is defined

then it should be scalar and of the same type as the variable. If the

variable is packed using scale_factor and add_offset attributes (see

below), the _FillValue attribute should have the data type of the

packed data.

It is not necessary to define your own _FillValue attribute for a

variable if the default fill value for the type of the variable is

adequate. However, use of the default fill value for data type byte is

not recommended. Note that if you change the value of this attribute,

the changed value applies only to subsequent writes; previously

written data are not changed.

Generic applications often need to write a value to represent

undefined or missing values. The fill value provides an appropriate

value for this purpose because it is normally outside the valid range

and therefore treated as missing when read by generic applications. It

is legal (but not recommended) for the fill value to be within the

valid range.

\section missing_value

This attribute is not treated in any special way by the library or

conforming generic applications, but is often useful documentation and

may be used by specific applications. The missing_value attribute can

be a scalar or vector containing values indicating missing data. These

values should all be outside the valid range so that generic

applications will treat them as missing.

When scale_factor and add_offset are used for packing, the value(s) of

the missing_value attribute should be specified in the domain of the

data in the file (the packed data), so that missing values can be

detected before the scale_factor and add_offset are applied.

valid_min A scalar specifying the minimum valid value for this

variable.  valid_max A scalar specifying the maximum valid value for

this variable.  valid_range A vector of two numbers specifying the

minimum and maximum valid values for this variable, equivalent to

specifying values for both valid_min and valid_max attributes. Any of

these attributes define the valid range. The attribute valid_range

must not be defined if either valid_min or valid_max is defined.

Generic applications should treat values outside the valid range as

missing. The type of each valid_range, valid_min and valid_max

attribute should match the type of its variable (except that for byte

data, these can be of a signed integral type to specify the intended

range).

If neither valid_min, valid_max nor valid_range is defined then

generic applications should define a valid range as follows. If the

data type is byte and _FillValue is not explicitly defined, then the

valid range should include all possible values. Otherwise, the valid

range should exclude the _FillValue (whether defined explicitly or by

default) as follows. If the _FillValue is positive then it defines a

valid maximum, otherwise it defines a valid minimum. For integer

types, there should be a difference of 1 between the _FillValue and

this valid minimum or maximum. For floating point types, the

difference should be twice the minimum possible (1 in the least

significant bit) to allow for rounding error.

If the variable is packed using scale_factor and add_offset attributes

(see below), the _FillValue, missing_value, valid_range, valid_min, or

valid_max attributes should have the data type of the packed data.

\section scale_factor

If present for a variable, the data are to be multiplied by this

factor after the data are read by the application that accesses the

data.

If valid values are specified using the valid_min, valid_max,

valid_range, or _FillValue attributes, those values should be

specified in the domain of the data in the file (the packed data), so

that they can be interpreted before the scale_factor and add_offset

are applied.

\section add_offset

If present for a variable, this number is to be added to the data

after it is read by the application that accesses the data. If both

scale_factor and add_offset attributes are present, the data are first

scaled before the offset is added. The attributes scale_factor and

add_offset can be used together to provide simple data compression to

store low-resolution floating-point data as small integers in a netCDF

dataset. When scaled data are written, the application should first

subtract the offset and then divide by the scale factor, rounding the

result to the nearest integer to avoid a bias caused by truncation

towards zero.

When scale_factor and add_offset are used for packing, the associated

variable (containing the packed data) is typically of type byte or

short, whereas the unpacked values are intended to be of type float or

double. The attributes scale_factor and add_offset should both be of

the type intended for the unpacked data, e.g. float or double.

\section coordinates

Following the CF (Climate and Forecast) conventions for netCDF

metadata, we define an <em>auxiliary coordinate variable</em> as any netCDF

variable that contains coordinate data, but is not a coordinate

variable (See \ref coordinate_variables).  Unlike coordinate

variables, there is no relationship between the name of an auxiliary

coordinate variable and the name(s) of its dimension(s).

The value of the coordinates attribute is a blank separated list of

names of auxiliary coordinate variables and (optionally) coordinate

variables.  There is no restriction on the order in which the variable

names appear in the coordinates attribute string.

\section signedness

Deprecated attribute, originally designed to indicate whether byte

values should be treated as signed or unsigned. The attributes

valid_min and valid_max may be used for this purpose. For example, if

you intend that a byte variable store only non-negative values, you

can use valid_min = 0 and valid_max = 255. This attribute is ignored

by the netCDF library.

\section C_format

A character array providing the format that should be used by C

applications to print values for this variable. For example, if you

know a variable is only accurate to three significant digits, it would

be appropriate to define the C_format attribute as "%.3g". The ncdump

utility program uses this attribute for variables for which it is

defined. The format applies to the scaled (internal) type and value,

regardless of the presence of the scaling attributes scale_factor and

add_offset.

\section FORTRAN_format

A character array providing the format that should be used by FORTRAN

applications to print values for this variable. For example, if you

know a variable is only accurate to three significant digits, it would

be appropriate to define the FORTRAN_format attribute as "(G10.3)".

\section title

A global attribute that is a character array providing a succinct

description of what is in the dataset.

\section history

A global attribute for an audit trail. This is a character array with

a line for each invocation of a program that has modified the

dataset. Well-behaved generic netCDF applications should append a line

containing: date, time of day, user name, program name and command

arguments.

\section Conventions

If present, 'Conventions' is a global attribute that is a character

array for the name of the conventions followed by the

dataset. Originally, these conventions were named by a string that was

interpreted as a directory name relative to the directory

/pub/netcdf/Conventions/ on the host ftp.unidata.ucar.edu. The web

page http://www.unidata.ucar.edu/netcdf/conventions.html is now the

preferred and authoritative location for registering a URI reference

to a set of conventions maintained elsewhere. The FTP site will be

preserved for compatibility with existing references, but authors of

new conventions should submit a request to

support-netcdf@unidata.ucar.edu for listing on the Unidata conventions

web page.

It may be convenient for defining institutions and groups to use a

hierarchical structure for general conventions and more specialized

conventions. For example, if a group named NUWG agrees upon a set of

conventions for dimension names, variable names, required attributes,

and netCDF representations for certain discipline-specific data

structures, they may store a document describing the agreed-upon

conventions in a dataset in the NUWG/ subdirectory of the Conventions

directory. Datasets that followed these conventions would contain a

global Conventions attribute with value "NUWG".

Later, if the group agrees upon some additional conventions for a

specific subset of NUWG data, for example time series data, the

description of the additional conventions might be stored in the

NUWG/Time_series/ subdirectory, and datasets that adhered to these

additional conventions would use the global Conventions attribute with

value "NUWG/Time_series", implying that this dataset adheres to the

NUWG conventions and also to the additional NUWG time-series

conventions.

It is possible for a netCDF file to adhere to more than one set of

conventions, even when there is no inheritance relationship among the

conventions. In this case, the value of the `Conventions' attribute

may be a single text string containing a list of the convention names

separated by blank space (recommended) or commas (if a convention name

contains blanks).

Typical conventions web sites will include references to documents in

some form agreed upon by the community that supports the conventions

and examples of netCDF file structures that follow the conventions.

\page background Background and Evolution of the NetCDF Interface

The development of the netCDF interface began with a modest goal

related to Unidata's needs: to provide a common interface between

Unidata applications and real-time meteorological data. Since Unidata

software was intended to run on multiple hardware platforms with

access from both C and FORTRAN, achieving Unidata's goals had the

potential for providing a package that was useful in a broader

context. By making the package widely available and collaborating with

other organizations with similar needs, we hoped to improve the then

current situation in which software for scientific data access was

only rarely reused by others in the same discipline and almost never

reused between disciplines (Fulker, 1988).

Important concepts employed in the netCDF software originated in a

paper (Treinish and Gough, 1987) that described data-access software

developed at the NASA Goddard National Space Science Data Center

(NSSDC). The interface provided by this software was called the Common

Data Format (CDF). The NASA CDF was originally developed as a

platform-specific FORTRAN library to support an abstraction for

storing arrays.

The NASA CDF package had been used for many different kinds of data in

an extensive collection of applications. It had the virtues of

simplicity (only 13 subroutines), independence from storage format,

generality, ability to support logical user views of data, and support

for generic applications.

Unidata held a workshop on CDF in Boulder in August 1987. We proposed

exploring the possibility of collaborating with NASA to extend the CDF

FORTRAN interface, to define a C interface, and to permit the access

of data aggregates with a single call, while maintaining compatibility

with the existing NASA interface.

Independently, Dave Raymond at the New Mexico Institute of Mining and

Technology had developed a package of C software for UNIX that

supported sequential access to self-describing array-oriented data and

a "pipes and filters" (or "data flow") approach to processing,

analyzing, and displaying the data. This package also used the "Common

Data Format" name, later changed to C-Based Analysis and Display

System (CANDIS). Unidata learned of Raymond's work (Raymond, 1988),

and incorporated some of his ideas, such as the use of named

dimensions and variables with differing shapes in a single data

object, into the Unidata netCDF interface.

In early 1988, Glenn Davis of Unidata developed a prototype netCDF

package in C that was layered on XDR. This prototype proved that a

single-file, XDR-based implementation of the CDF interface could be

achieved at acceptable cost and that the resulting programs could be

implemented on both UNIX and VMS systems. However, it also

demonstrated that providing a small, portable, and NASA CDF-compatible

FORTRAN interface with the desired generality was not

practical. NASA's CDF and Unidata's netCDF have since evolved

separately, but recent CDF versions share many characteristics with

netCDF.

In early 1988, Joe Fahle of SeaSpace, Inc. (a commercial software

development firm in San Diego, California), a participant in the 1987

Unidata CDF workshop, independently developed a CDF package in C that

extended the NASA CDF interface in several important ways (Fahle,

1989). Like Raymond's package, the SeaSpace CDF software permitted

variables with unrelated shapes to be included in the same data object

and permitted a general form of access to multidimensional

arrays. Fahle's implementation was used at SeaSpace as the

intermediate form of storage for a variety of steps in their

image-processing system. This interface and format have subsequently

evolved into the Terascan data format.

After studying Fahle's interface, we concluded that it solved many of

the problems we had identified in trying to stretch the NASA interface

to our purposes. In August 1988, we convened a small workshop to agree

on a Unidata netCDF interface, and to resolve remaining open

issues. Attending were Joe Fahle of SeaSpace, Michael Gough of Apple

(an author of the NASA CDF software), Angel Li of the University of

Miami (who had implemented our prototype netCDF software on VMS and

was a potential user), and Unidata systems development

staff. Consensus was reached at the workshop after some further

simplifications were discovered. A document incorporating the results

of the workshop into a proposed Unidata netCDF interface specification

was distributed widely for comments before Glenn Davis and Russ Rew

implemented the first version of the software. Comparison with other

data-access interfaces and experience using netCDF are discussed in

Rew and Davis (1990a), Rew and Davis (1990b), Jenter and Signell

(1992), and Brown, Folk, Goucher, and Rew (1993).

In October 1991, we announced version 2.0 of the netCDF software

distribution. Slight modifications to the C interface (declaring

dimension lengths to be long rather than int) improved the usability

of netCDF on inexpensive platforms such as MS-DOS computers, without

requiring recompilation on other platforms. This change to the

interface required no changes to the associated file format.

Release of netCDF version 2.3 in June 1993 preserved the same file

format but added single call access to records, optimizations for

accessing cross-sections involving non-contiguous data, subsampling

along specified dimensions (using 'strides'), accessing non-contiguous

data (using 'mapped array sections'), improvements to the ncdump and

ncgen utilities, and an experimental C++ interface.

In version 2.4, released in February 1996, support was added for new

platforms and for the C++ interface, significant optimizations were

implemented for supercomputer architectures, and the file format was

formally specified in an appendix to the User's Guide.

FAN (File Array Notation), software providing a high-level interface

to netCDF data, was made available in May 1996. The capabilities of

the FAN utilities include extracting and manipulating array data from

netCDF datasets, printing selected data from netCDF arrays, copying

ASCII data into netCDF arrays, and performing various operations (sum,

mean, max, min, product, and others) on netCDF arrays.

In 1996 and 1997, Joe Sirott implemented and made available the first

implementation of a read-only netCDF interface for Java, Bill Noon

made a Python module available for netCDF, and Konrad Hinsen

contributed another netCDF interface for Python.

In May 1997, Version 3.3 of netCDF was released. This included a new

type-safe interface for C and Fortran, as well as many other

improvements. A month later, Charlie Zender released version 1.0 of

the NCO (netCDF Operators) package, providing command-line utilities

for general purpose operations on netCDF data.

Version 3.4 of Unidata's netCDF software, released in March 1998,

included initial large file support, performance enhancements, and

improved Cray platform support. Later in 1998, Dan Schmitt provided a

Tcl/Tk interface, and Glenn Davis provided version 1.0 of netCDF for

Java.

In May 1999, Glenn Davis, who was instrumental in creating and

developing netCDF, died in a small plane crash during a

thunderstorm. The memory of Glenn's passions and intellect continue to

inspire those of us who worked with him.

In February 2000, an experimental Fortran 90 interface developed by

Robert Pincus was released.

John Caron released netCDF for Java, version 2.0 in February

2001. This version incorporated a new high-performance package for

multidimensional arrays, simplified the interface, and included

OpenDAP (known previously as DODS) remote access, as well as remote

netCDF access via HTTP contributed by Don Denbo.

In March 2001, netCDF 3.5.0 was released. This release fully

integrated the new Fortran 90 interface, enhanced portability,

improved the C++ interface, and added a few new tuning functions.

Also in 2001, Takeshi Horinouchi and colleagues made a netCDF

interface for Ruby available, as did David Pierce for the R language

for statistical computing and graphics. Charles Denham released

WetCDF, an independent implementation of the netCDF interface for

Matlab, as well as updates to the popular netCDF Toolbox for Matlab.

In 2002, Unidata and collaborators developed NcML, an XML

representation for netCDF data useful for cataloging data holdings,

aggregation of data from multiple datasets, augmenting metadata in

existing datasets, and support for alternative views of data. The Java

interface currently provides access to netCDF data through NcML.

Additional developments in 2002 included translation of C and Fortran

User Guides into Japanese by Masato Shiotani and colleagues, creation

of a “Best Practices” guide for writing netCDF files, and provision of

an Ada-95 interface by Alexandru Corlan.

In July 2003 a group of researchers at Northwestern University and

Argonne National Laboratory (Jianwei Li, Wei-keng Liao, Alok

Choudhary, Robert Ross, Rajeev Thakur, William Gropp, and Rob Latham)

contributed a new parallel interface for writing and reading netCDF

data, tailored for use on high performance platforms with parallel

I/O. The implementation built on the MPI-IO interface, providing

portability to many platforms.

In October 2003, Greg Sjaardema contributed support for an alternative

format with 64-bit offsets, to provide more complete support for very

large files. These changes, with slight modifications at Unidata, were

incorporated into version 3.6.0, released in December, 2004.

In 2004, thanks to a NASA grant, Unidata and NCSA began a

collaboration to increase the interoperability of netCDF and HDF5, and

bring some advanced HDF5 features to netCDF users.

In February, 2006, release 3.6.1 fixed some minor bugs.

In March, 2007, release 3.6.2 introduced an improved build system that

used automake and libtool, and an upgrade to the most recent autoconf

release, to support shared libraries and the netcdf-4 builds. This

release also introduced the NetCDF Tutorial and example programs.

The first beta release of netCDF-4.0 was celebrated with a giant party

at Unidata in April, 2007. Over 2000 people danced 'til dawn at the

NCAR Mesa Lab, listening to the Flaming Lips and the Denver Gilbert &

Sullivan repertory company. Brittany Spears performed the

world-premire of her smash hit "Format me baby, one more time."

In June, 2008, netCDF-4.0 was released. Version 3.6.3, the same code

but with netcdf-4 features turned off, was released at the same

time. The 4.0 release uses HDF5 1.8.1 as the data storage layer for

netcdf, and introduces many new features including groups and

user-defined types. The 3.6.3/4.0 releases also introduced handling of

UTF8-encoded Unicode names.

NetCDF-4.1.1 was released in April, 2010, provided built-in client

support for the DAP protocol for accessing data from remote OPeNDAP

servers, full support for the enhanced netCDF-4 data model in the

ncgen utility, a new nccopy utility for copying and conversion among

netCDF format variants, ability to read some HDF4/HDF5 data archives

through the netCDF C or Fortran interfaces, support for parallel I/O

on netCDF classic and 64-bit offset files using the parallel-netcdf

(formerly pnetcdf) library from Argonne/Northwestern, a new nc-config

utility to help compile and link programs that use netCDF, inclusion

of the UDUNITS library for hadling “units” attributes, and inclusion

of libcf to assist in creating data compliant with the Climate and

Forecast (CF) metadata conventions.

In September, 2010, the Netcdf-Java/CDM (Common Data Model) version

4.2 library was declared stable and made available to users. This

100%-Java implementation provides a read-write interface to netCDF-3

classic and 64-bit offset data, as well as a read-onlt interface to

netCDF-4 enhanced model data and many other formats of scientific data

through a common (CDM) interface. The NetCDF-Java library also

implements NcML, which allows you to add metadata to CDM datasets, as

well as to create virtual datasets through aggregation. A ToolsUI

application is also included that provides a graphical user interface

to capabilities similar to the C-based ncdump and ncgen utilities, as

well as CF-compliance checking and many other features.

\page remote_client The Remote Data Access Client

Starting with version 4.1.1 the netCDF C libraries and utilities have

supported remote data access.

\page data_access Data Access

To access (read or write) netCDF data you specify an open netCDF

dataset, a netCDF variable, and information (e.g., indices)

identifying elements of the variable. The name of the access function

corresponds to the internal type of the data. If the internal type has

a different representation from the external type of the variable, a

conversion between the internal type and external type will take place

when the data is read or written.

Access to data in classic and 64-bit offset format is direct. Access

to netCDF-4 data is buffered by the HDF5 layer. In either case you can

access a small subset of data from a large dataset efficiently,

without first accessing all the data that precedes it.

Reading and writing data by specifying a variable, instead of a

position in a file, makes data access independent of how many other

variables are in the dataset, making programs immune to data format

changes that involve adding more variables to the data.

In the C and FORTRAN interfaces, datasets are not specified by name

every time you want to access data, but instead by a small integer

called a dataset ID, obtained when the dataset is first created or

opened.

Similarly, a variable is not specified by name for every data access

either, but by a variable ID, a small integer used to identify each

variable in a netCDF dataset.

\section forms_of_data_access Forms of Data Access

The netCDF interface supports several forms of direct access to data

values in an open netCDF dataset. We describe each of these forms of

access in order of increasing generality:

- access to all elements;

- access to individual elements, specified with an index vector;

- access to array sections, specified with an index vector, and count vector;

- access to sub-sampled array sections, specified with an index

vector, count vector, and stride vector; and

- access to mapped array sections, specified with an index vector,

count vector, stride vector, and an index mapping vector.

The four types of vector (index vector, count vector, stride vector

and index mapping vector) each have one element for each dimension of

the variable. Thus, for an n-dimensional variable (rank = n),

n-element vectors are needed. If the variable is a scalar (no

dimensions), these vectors are ignored.

An array section is a "slab" or contiguous rectangular block that is

specified by two vectors. The index vector gives the indices of the

element in the corner closest to the origin. The count vector gives

the lengths of the edges of the slab along each of the variable's

dimensions, in order. The number of values accessed is the product of

these edge lengths.

A subsampled array section is similar to an array section, except that

an additional stride vector is used to specify sampling. This vector

has an element for each dimension giving the length of the strides to

be taken along that dimension. For example, a stride of 4 means every

fourth value along the corresponding dimension. The total number of

values accessed is again the product of the elements of the count

vector.

A mapped array section is similar to a subsampled array section except

that an additional index mapping vector allows one to specify how data

values associated with the netCDF variable are arranged in memory. The

offset of each value from the reference location, is given by the sum

of the products of each index (of the imaginary internal array which

would be used if there were no mapping) by the corresponding element

of the index mapping vector. The number of values accessed is the same

as for a subsampled array section.

The use of mapped array sections is discussed more fully below, but

first we present an example of the more commonly used array-section

access.

\section c_array_section_access A C Example of Array-Section Access

Assume that in our earlier example of a netCDF dataset (see Network

Common Data Form Language (CDL)), we wish to read a cross-section of

all the data for the temp variable at one level (say, the second), and

assume that there are currently three records (time values) in the

netCDF dataset. Recall that the dimensions are defined as

\code

lat = 5, lon = 10, level = 4, time = unlimited;

\endcode

and the variable temp is declared as

\code

float   temp(time, level, lat, lon);

\endcode

in the CDL notation.

A corresponding C variable that holds data for only one level might be

declared as:

\code

#define LATS  5

#define LONS 10

#define LEVELS 1

#define TIMES 3                 /* currently */

...

float   temp[TIMES*LEVELS*LATS*LONS];

\endcode

to keep the data in a one-dimensional array, or

\code

...

float   temp[TIMES][LEVELS][LATS][LONS];

\endcode

using a multidimensional array declaration.

To specify the block of data that represents just the second level,

all times, all latitudes, and all longitudes, we need to provide a

start index and some edge lengths. The start index should be (0, 1, 0,

0) in C, because we want to start at the beginning of each of the

time, lon, and lat dimensions, but we want to begin at the second

value of the level dimension. The edge lengths should be (3, 1, 5, 10)

in C, (since we want to get data for all three time values, only one

level value, all five lat values, and all 10 lon values. We should

expect to get a total of 150 floating-point values returned (3 * 1 * 5

* 10), and should provide enough space in our array for this many. The

order in which the data will be returned is with the last dimension,

lon, varying fastest:

\code

temp[0][1][0][0]

temp[0][1][0][1]

temp[0][1][0][2]

temp[0][1][0][3]

...

temp[2][1][4][7]

temp[2][1][4][8]

temp[2][1][4][9]

\endcode

Different dimension orders for the C, FORTRAN, or other language

interfaces do not reflect a different order for values stored on the

disk, but merely different orders supported by the procedural

interfaces to the languages. In general, it does not matter whether a

netCDF dataset is written using the C, FORTRAN, or another language

interface; netCDF datasets written from any supported language may be

read by programs written in other supported languages.  3.4.3 More on

General Array Section Access for C

The use of mapped array sections allows non-trivial relationships

between the disk addresses of variable elements and the addresses

where they are stored in memory. For example, a matrix in memory could

be the transpose of that on disk, giving a quite different order of

elements. In a regular array section, the mapping between the disk and

memory addresses is trivial: the structure of the in-memory values

(i.e., the dimensional lengths and their order) is identical to that

of the array section. In a mapped array section, however, an index

mapping vector is used to define the mapping between indices of netCDF

variable elements and their memory addresses.

With mapped array access, the offset (number of array elements) from

the origin of a memory-resident array to a particular point is given

by the inner product[1] of the index mapping vector with the point's

coordinate offset vector. A point's coordinate offset vector gives,

for each dimension, the offset from the origin of the containing array

to the point.In C, a point's coordinate offset vector is the same as

its coordinate vector.

The index mapping vector for a regular array section would have–in

order from most rapidly varying dimension to most slowly–a constant 1,

the product of that value with the edge length of the most rapidly

varying dimension of the array section, then the product of that value

with the edge length of the next most rapidly varying dimension, and

so on. In a mapped array, however, the correspondence between netCDF

variable disk locations and memory locations can be different.

For example, the following C definitions:

\code

struct vel {

int flags;

float u;

float v;

} vel[NX][NY];

ptrdiff_t imap[2] = {

sizeof(struct vel),

sizeof(struct vel)*NY

};

\endcode

where imap is the index mapping vector, can be used to access the

memory-resident values of the netCDF variable, vel(NY,NX), even though

the dimensions are transposed and the data is contained in a 2-D array

of structures rather than a 2-D array of floating-point values.

A detailed example of mapped array access is presented in the

description of the interfaces for mapped array access. See Write a

Mapped Array of Values - nc_put_varm_ type.

Note that, although the netCDF abstraction allows the use of

subsampled or mapped array-section access there use is not

required. If you do not need these more general forms of access, you

may ignore these capabilities and use single value access or regular

array section access instead.

\page dimensions Dimensions

A dimension may be used to represent a real physical dimension, for

example, time, latitude, longitude, or height. A dimension might also

be used to index other quantities, for example station or

model-run-number.

A netCDF dimension has both a name and a length.

A dimension length is an arbitrary positive integer, except that one

dimension in a classic or 64-bit offset netCDF dataset can have the

length UNLIMITED. In a netCDF-4 dataset, any number of unlimited

dimensions can be used.

Such a dimension is called the unlimited dimension or the record

dimension. A variable with an unlimited dimension can grow to any

length along that dimension. The unlimited dimension index is like a

record number in conventional record-oriented files.

A netCDF classic or 64-bit offset dataset can have at most one

unlimited dimension, but need not have any. If a variable has an

unlimited dimension, that dimension must be the most significant

(slowest changing) one. Thus any unlimited dimension must be the first

dimension in a CDL shape and the first dimension in corresponding C

array declarations.

A netCDF-4 dataset may have multiple unlimited dimensions, and there

are no restrictions on their order in the list of a variables

dimensions.

To grow variables along an unlimited dimension, write the data using

any of the netCDF data writing functions, and specify the index of the

unlimited dimension to the desired record number. The netCDF library

will write however many records are needed (using the fill value,

unless that feature is turned off, to fill in any intervening

records).

CDL dimension declarations may appear on one or more lines following

the CDL keyword dimensions. Multiple dimension declarations on the

same line may be separated by commas. Each declaration is of the form

name = length. Use the “/” character to include group information

(netCDF-4 output only).

There are four dimensions in the above example: lat, lon, level, and

time (see \ref data_model). The first three are assigned fixed

lengths; time is assigned the length UNLIMITED, which means it is the

unlimited dimension.

The basic unit of named data in a netCDF dataset is a variable. When a

variable is defined, its shape is specified as a list of

dimensions. These dimensions must already exist. The number of

dimensions is called the rank (a.k.a. dimensionality). A scalar

variable has rank 0, a vector has rank 1 and a matrix has rank 2.

It is possible (since version 3.1 of netCDF) to use the same dimension

more than once in specifying a variable shape. For example,

correlation(instrument, instrument) could be a matrix giving

correlations between measurements using different instruments. But

data whose dimensions correspond to those of physical space/time

should have a shape comprising different dimensions, even if some of

these have the same length.

\page variables Variables

Variables are used to store the bulk of the data in a netCDF

dataset. A variable represents an array of values of the same type. A

scalar value is treated as a 0-dimensional array. A variable has a

name, a data type, and a shape described by its list of dimensions

specified when the variable is created. A variable may also have

associated attributes, which may be added, deleted or changed after

the variable is created.

A variable external data type is one of a small set of netCDF

types. In classic and 64-bit offset files, only the original six types

are available (byte, character, short, int, float, and

double). Variables in netCDF-4 files may also use unsigned short,

unsigned int, 64-bit int, unsigned 64-bit int, or string. Or the user

may define a type, as an opaque blob of bytes, as an array of variable

length arrays, or as a compound type, which acts like a C struct. (See

\ref data_type).

In the CDL notation, classic and 64-bit offset type can be used. They

are given the simpler names byte, char, short, int, float, and

double. The name real may be used as a synonym for float in the CDL

notation. The name long is a deprecated synonym for int. For the exact

meaning of each of the types see External Types. The ncgen utility

supports new primitive types with names ubyte, ushort, uint, int64,

uint64, and string.

CDL variable declarations appear after the variable keyword in a CDL

unit. They have the form

\code

type variable_name ( dim_name_1, dim_name_2, ... );

\endcode

for variables with dimensions, or

\code

type variable_name;

\endcode

for scalar variables.

In the above CDL example there are six variables. As discussed below,

four of these are coordinate variables (See coordinate_variables). The

remaining variables (sometimes called primary variables), temp and rh,

contain what is usually thought of as the data. Each of these

variables has the unlimited dimension time as its first dimension, so

they are called record variables. A variable that is not a record

variable has a fixed length (number of data values) given by the

product of its dimension lengths. The length of a record variable is

also the product of its dimension lengths, but in this case the

product is variable because it involves the length of the unlimited

dimension, which can vary. The length of the unlimited dimension is

the number of records.

\page coordinate_variables Coordinate Variables

It is legal for a variable to have the same name as a dimension. Such

variables have no special meaning to the netCDF library. However there

is a convention that such variables should be treated in a special way

by software using this library.

A variable with the same name as a dimension is called a

<em>coordinate variable</em>. It typically defines a physical coordinate corresponding to

that dimension. The above CDL example includes the coordinate

variables lat, lon, level and time, defined as follows:

\code

int     lat(lat), lon(lon), level(level);

short   time(time);

...

data:

level   = 1000, 850, 700, 500;

lat     = 20, 30, 40, 50, 60;

lon     = -160,-140,-118,-96,-84,-52,-45,-35,-25,-15;

time    = 12;

\endcode

These define the latitudes, longitudes, barometric pressures and times

corresponding to positions along these dimensions. Thus there is data

at altitudes corresponding to 1000, 850, 700 and 500 millibars; and at

latitudes 20, 30, 40, 50 and 60 degrees north. Note that each

coordinate variable is a vector and has a shape consisting of just the

dimension with the same name.

A position along a dimension can be specified using an index. This is

an integer with a minimum value of 0 for C programs, 1 in Fortran

programs. Thus the 700 millibar level would have an index value of 2

in the example above in a C program, and 3 in a Fortran program.

If a dimension has a corresponding coordinate variable, then this

provides an alternative, and often more convenient, means of

specifying position along it. Current application packages that make

use of coordinate variables commonly assume they are numeric vectors

and strictly monotonic (all values are different and either increasing

or decreasing).

\page attributes Attributes

NetCDF attributes are used to store data about the data (ancillary

data or metadata), similar in many ways to the information stored in

data dictionaries and schema in conventional database systems. Most

attributes provide information about a specific variable. These are

identified by the name (or ID) of that variable, together with the

name of the attribute.

Some attributes provide information about the dataset as a whole and

are called global attributes. These are identified by the attribute

name together with a blank variable name (in CDL) or a special null

"global variable" ID (in C or Fortran).

In netCDF-4 file, attributes can also be added at the group level.

An attribute has an associated variable (the null "global variable"

for a global or group-level attribute), a name, a data type, a length,

and a value. The current version treats all attributes as vectors;

scalar values are treated as single-element vectors.

Conventional attribute names should be used where applicable. New

names should be as meaningful as possible.

The external type of an attribute is specified when it is created. The

types permitted for attributes are the same as the netCDF external

data types for variables. Attributes with the same name for different

variables should sometimes be of different types. For example, the

attribute valid_max specifying the maximum valid data value for a

variable of type int should be of type int, whereas the attribute

valid_max for a variable of type double should instead be of type

double.

Attributes are more dynamic than variables or dimensions; they can be

deleted and have their type, length, and values changed after they are

created, whereas the netCDF interface provides no way to delete a

variable or to change its type or shape.

The CDL notation for defining an attribute is

\code

variable_name:attribute_name = list_of_values;

\endcode

for a variable attribute, or

\code

:attribute_name = list_of_values;

\endcode

for a global attribute.

For the netCDF classic model, the type and length of each attribute

are not explicitly declared in CDL; they are derived from the values

assigned to the attribute. All values of an attribute must be of the

same type. The notation used for constant values of the various netCDF

types is discussed later (see CDL Constants).

The extended CDL syntax for the enhanced data model supported by

netCDF-4 allows optional type specifications, including user-defined

types, for attributes of user-defined types. See ncdump output or the

reference documentation for ncgen for details of the extended CDL

systax.

In the netCDF example (see \ref data_model), units is an attribute for

the variable lat that has a 13-character array value

'degrees_north'. And valid_range is an attribute for the variable rh

that has length 2 and values '0.0' and '1.0'.

One global attribute, called “source”, is defined for the example

netCDF dataset. This is a character array intended for documenting the

data. Actual netCDF datasets might have more global attributes to

document the origin, history, conventions, and other characteristics

of the dataset as a whole.

Most generic applications that process netCDF datasets assume standard

attribute conventions and it is strongly recommended that these be

followed unless there are good reasons for not doing so. For

information about units, long_name, valid_min, valid_max, valid_range,

scale_factor, add_offset, _FillValue, and other conventional

attributes, see Attribute Conventions.

Attributes may be added to a netCDF dataset long after it is first

defined, so you don't have to anticipate all potentially useful

attributes. However adding new attributes to an existing classic or

64-bit offset format dataset can incur the same expense as copying the

dataset. For a more extensive discussion see Structure.

\page differences_atts_vars Differences between Attributes and Variables

In contrast to variables, which are intended for bulk data, attributes

are intended for ancillary data, or information about the data. The

total amount of ancillary data associated with a netCDF object, and

stored in its attributes, is typically small enough to be

memory-resident. However variables are often too large to entirely fit

in memory and must be split into sections for processing.

Another difference between attributes and variables is that variables

may be multidimensional. Attributes are all either scalars

(single-valued) or vectors (a single, fixed dimension).

Variables are created with a name, type, and shape before they are

assigned data values, so a variable may exist with no values. The

value of an attribute is specified when it is created, unless it is a

zero-length attribute.

A variable may have attributes, but an attribute cannot have

attributes. Attributes assigned to variables may have the same units

as the variable (for example, valid_range) or have no units (for

example, scale_factor). If you want to store data that requires units

different from those of the associated variable, it is better to use a

variable than an attribute. More generally, if data require ancillary

data to describe them, are multidimensional, require any of the

defined netCDF dimensions to index their values, or require a

significant amount of storage, that data should be represented using

variables rather than attributes.

\page classic_file_parts Parts of a NetCDF Classic File

A netCDF classic or 64-bit offset dataset is stored as a single file

comprising two parts:

- a header, containing all the information about dimensions, attributes,

and variables except for the variable data;

- a data part, comprising fixed-size data, containing the data for

variables that don't have an unlimited dimension; and variable-size

data, containing the data for variables that have an unlimited

dimension.

Both the header and data parts are represented in a

machine-independent form. This form is very similar to XDR (eXternal

Data Representation), extended to support efficient storage of arrays

of non-byte data.

The header at the beginning of the file contains information about the

dimensions, variables, and attributes in the file, including their

names, types, and other characteristics. The information about each

variable includes the offset to the beginning of the variable's data

for fixed-size variables or the relative offset of other variables

within a record. The header also contains dimension lengths and

information needed to map multidimensional indices for each variable

to the appropriate offsets.

By default, this header has little usable extra space; it is only as

large as it needs to be for the dimensions, variables, and attributes

(including all the attribute values) in the netCDF dataset, with a

small amount of extra space from rounding up to the nearest disk block

size. This has the advantage that netCDF files are compact, requiring

very little overhead to store the ancillary data that makes the

datasets self-describing. A disadvantage of this organization is that

any operation on a netCDF dataset that requires the header to grow

(or, less likely, to shrink), for example adding new dimensions or new

variables, requires moving the data by copying it. This expense is

incurred when the enddef function is called: nc_enddef in C (see

nc_enddef), NF_ENDDEF in Fortran (see NF_ENDDEF), after a previous

call to the redef function: nc_redef in C (see nc_redef) or NF_REDEF

in Fortran (see NF_REDEF). If you create all necessary dimensions,

variables, and attributes before writing data, and avoid later

additions and renamings of netCDF components that require more space

in the header part of the file, you avoid the cost associated with

later changing the header.

Alternatively, you can use an alternative version of the enddef

function with two underbar characters instead of one to explicitly

reserve extra space in the file header when the file is created: in C

nc__enddef (see nc__enddef), in Fortran NF__ENDDEF (see NF__ENDDEF),

after a previous call to the redef function. This avoids the expense

of moving all the data later by reserving enough extra space in the

header to accommodate anticipated changes, such as the addition of new

attributes or the extension of existing string attributes to hold

longer strings.

When the size of the header is changed, data in the file is moved, and

the location of data values in the file changes. If another program is

reading the netCDF dataset during redefinition, its view of the file

will be based on old, probably incorrect indexes. If netCDF datasets

are shared across redefinition, some mechanism external to the netCDF

library must be provided that prevents access by readers during

redefinition, and causes the readers to call nc_sync/NF_SYNC before

any subsequent access.

The fixed-size data part that follows the header contains all the

variable data for variables that do not employ an unlimited

dimension. The data for each variable is stored contiguously in this

part of the file. If there is no unlimited dimension, this is the last

part of the netCDF file.

The record-data part that follows the fixed-size data consists of a

variable number of fixed-size records, each of which contains data for

all the record variables. The record data for each variable is stored

contiguously in each record.

The order in which the variable data appears in each data section is

the same as the order in which the variables were defined, in

increasing numerical order by netCDF variable ID. This knowledge can

sometimes be used to enhance data access performance, since the best

data access is currently achieved by reading or writing the data in

sequential order.

\page parts_of_netcdf4 Parts of a NetCDF-4 HDF5 File

NetCDF-4 files are created with the HDF5 library, and are HDF5 files

in every way, and can be read without the netCDF-4 interface. (Note

that modifying these files with HDF5 will almost certainly make them

unreadable to netCDF-4.)

Groups in a netCDF-4 file correspond with HDF5 groups (although the

netCDF-4 tree is rooted not at the HDF5 root, but in group “_netCDF”).

Variables in netCDF coorespond with identically named datasets in

HDF5. Attributes similarly.

Since there is more metadata in a netCDF file than an HDF5 file,

special datasets are used to hold netCDF metadata.

The _netcdf_dim_info dataset (in group _netCDF) contains the ids of

the shared dimensions, and their length (0 for unlimited dimensions).

The _netcdf_var_info dataset (in group _netCDF) holds an array of

compound types which contain the variable ID, and the associated

dimension ids.

\page xdr_layer The Extended XDR Layer

XDR is a standard for describing and encoding data and a library of

functions for external data representation, allowing programmers to

encode data structures in a machine-independent way. Classic or 64-bit

offset netCDF employs an extended form of XDR for representing

information in the header part and the data parts. This extended XDR

is used to write portable data that can be read on any other machine

for which the library has been implemented.

The cost of using a canonical external representation for data varies

according to the type of data and whether the external form is the

same as the machine's native form for that type.

For some data types on some machines, the time required to convert

data to and from external form can be significant. The worst case is

reading or writing large arrays of floating-point data on a machine

that does not use IEEE floating-point as its native representation.

\page large_file_support Large File Support

It is possible to write netCDF files that exceed 2 GiByte on platforms

that have "Large File Support" (LFS). Such files are

platform-independent to other LFS platforms, but trying to open them

on an older platform without LFS yields a "file too large" error.

Without LFS, no files larger than 2 GiBytes can be used. The rest of

this section applies only to systems with LFS.

The original binary format of netCDF (classic format) limits the size

of data files by using a signed 32-bit offset within its internal

structure. Files larger than 2 GiB can be created, with certain

limitations. See Classic Limitations.

In version 3.6.0, netCDF included its first-ever variant of the

underlying data format. The new format introduced in 3.6.0 uses 64-bit

file offsets in place of the 32-bit offsets. There are still some

limits on the sizes of variables, but the new format can create very

large datasets. See 64 bit Offset Limitations.

NetCDF-4 variables and files can be any size supported by the

underlying file system.

The original data format (netCDF classic), is still the default data

format for the netCDF library.

The following table summarizes the size limitations of various

permutations of LFS support, netCDF version, and data format. Note

that 1 GiB = 2^30 bytes or about 1.07e+9 bytes, 1 EiB = 2^60 bytes or

about 1.15e+18 bytes. Note also that all sizes are really 4 bytes less

than the ones given below. For example the maximum size of a fixed

variable in netCDF 3.6 classic format is really 2 GiB - 4 bytes.

Limit 	No LFS 	v3.5 	v3.6/classic 	v3.6/64-bit offset 	v4.0/netCDF-4

Max File Size 	2 GiB 	8 EiB 	8 EiB 	8 EiB 	??

Max Number of Fixed Vars > 2 GiB 	0 	1 (last) 	1 (last) 	2^32 	??

Max Record Vars w/ Rec Size > 2 GiB 	0 	1 (last) 	1 (last) 	2^32 	??

Max Size of Fixed/Record Size of Record Var 	2 GiB 	2 GiB 	2 GiB 	4 GiB 	??

Max Record Size 	2 GiB/nrecs 	4 GiB 	8 EiB/nrecs 	8 EiB/nrecs 	??

For more information about the different file formats of netCDF See

Which Format.

\page offset_format_limitations NetCDF 64-bit Offset Format Limitations

Although the 64-bit offset format allows the creation of much larger

netCDF files than was possible with the classic format, there are

still some restrictions on the size of variables.

It's important to note that without Large File Support (LFS) in the

operating system, it's impossible to create any file larger than 2

GiBytes. Assuming an operating system with LFS, the following

restrictions apply to the netCDF 64-bit offset format.

No fixed-size variable can require more than 2^32 - 4 bytes (i.e. 4GiB

- 4 bytes, or 4,294,967,292 bytes) of storage for its data, unless it

is the last fixed-size variable and there are no record

variables. When there are no record variables, the last fixed-size

variable can be any size supported by the file system, e.g. terabytes.

A 64-bit offset format netCDF file can have up to 2^32 - 1 fixed sized

variables, each under 4GiB in size. If there are no record variables

in the file the last fixed variable can be any size.

No record variable can require more than 2^32 - 4 bytes of storage for

each record's worth of data, unless it is the last record variable. A

64-bit offset format netCDF file can have up to 2^32 - 1 records, of

up to 2^32 - 1 variables, as long as the size of one record's data for

each record variable except the last is less than 4 GiB - 4.

Note also that all netCDF variables and records are padded to 4 byte

boundaries.

\page classic_format_limitations NetCDF Classic Format Limitations

There are important constraints on the structure of large netCDF

classic files that result from the 32-bit relative offsets that are

part of the netCDF classic file format:

The maximum size of a record in the classic format in versions 3.5.1

and earlier is 2^32 - 4 bytes, or about 4 GiB. In versions 3.6.0 and

later, there is no such restriction on total record size for the

classic format or 64-bit offset format.

If you don't use the unlimited dimension, only one variable can exceed

2 GiB in size, but it can be as large as the underlying file system

permits. It must be the last variable in the dataset, and the offset

to the beginning of this variable must be less than about 2 GiB.

The limit is really 2^31 - 4. If you were to specify a variable size

of 2^31 -3, for example, it would be rounded up to the nearest

multiple of 4 bytes, which would be 2^31, which is larger than the

largest signed integer, 2^31 - 1.

For example, the structure of the data might be something like:

\code

netcdf bigfile1 {

dimensions:

x=2000;

y=5000;

z=10000;

variables:

double x(x);         // coordinate variables

double y(y);

double z(z);

double var(x, y, z); // 800 Gbytes

}

\endcode

If you use the unlimited dimension, record variables may exceed 2 GiB

in size, as long as the offset of the start of each record variable

within a record is less than 2 GiB - 4. For example, the structure of

the data in a 2.4 Tbyte file might be something like:

\code

netcdf bigfile2 {

dimensions:

x=2000;

y=5000;

z=10;

t=UNLIMITED;         // 1000 records, for example

variables:

double x(x);         // coordinate variables

double y(y);

double z(z);

double t(t);

// 3 record variables, 2400000000 bytes per record

double var1(t, x, y, z);

double var2(t, x, y, z);

double var3(t, x, y, z);

}

\endcode

\page netcdf_3_io The NetCDF-3 I/O Layer

The following discussion applies only to netCDF classic and 64-bit

offset files. For netCDF-4 files, the I/O layer is the HDF5 library.

For netCDF classic and 64-bit offset files, an I/O layer implemented

much like the C standard I/O (stdio) library is used by netCDF to read

and write portable data to netCDF datasets. Hence an understanding of

the standard I/O library provides answers to many questions about

multiple processes accessing data concurrently, the use of I/O

buffers, and the costs of opening and closing netCDF files. In

particular, it is possible to have one process writing a netCDF

dataset while other processes read it.

Data reads and writes are no more atomic than calls to stdio fread()

and fwrite(). An nc_sync/NF_SYNC call is analogous to the fflush call

in the C standard I/O library, writing unwritten buffered data so

other processes can read it; The C function nc_sync (see nc_sync), or

the Fortran function NF_SYNC (see NF_SYNC), also brings header changes

up-to-date (for example, changes to attribute values). Opening the

file with the NC_SHARE (in C) or the NF_SHARE (in Fortran) is

analogous to setting a stdio stream to be unbuffered with the _IONBF

flag to setvbuf.

As in the stdio library, flushes are also performed when "seeks" occur

to a different area of the file. Hence the order of read and write

operations can influence I/O performance significantly. Reading data

in the same order in which it was written within each record will

minimize buffer flushes.

You should not expect netCDF classic or 64-bit offset format data

access to work with multiple writers having the same file open for

writing simultaneously.

It is possible to tune an implementation of netCDF for some platforms

by replacing the I/O layer with a different platform-specific I/O

layer. This may change the similarities between netCDF and standard

I/O, and hence characteristics related to data sharing, buffering, and

the cost of I/O operations.

The distributed netCDF implementation is meant to be

portable. Platform-specific ports that further optimize the

implementation for better I/O performance are practical in some cases.

\page parallel_access Parallel Access with NetCDF-4

Use the special parallel open (or create) calls to open (or create) a

file, and then to use parallel I/O to read or write that file (see

nc_open_par()).

Note that the chunk cache is turned off if a file is opened for

parallel I/O in read/write mode. Open the file in read-only mode to

engage the chunk cache.

NetCDF uses the HDF5 parallel programming model for parallel I/O with

netCDF-4/HDF5 files. The HDF5 tutorial

(http://hdfgroup.org/HDF5//HDF5/Tutor) is a good reference.

For classic and 64-bit offset files, netCDF uses the parallel-netcdf

(formerly pnetcdf) library from Argonne National Labs/Nortwestern

University. For parallel access of classic and 64-bit offset files,

netCDF must be configured with the –with-pnetcdf option at build

time. See the parallel-netcdf site for more information

(http://www.mcs.anl.gov/parallel-netcdf).

\page interoperability_with_hdf5 Interoperability with HDF5

To create HDF5 files that can be read by netCDF-4, use the latest in

the HDF5 1.8.x series.

HDF5 has some features that will not be supported by netCDF-4, and

will cause problems for interoperability:

- HDF5 allows a Group to be both an ancestor and a descendant of

another Group, creating cycles in the subgroup graph. HDF5 also

permits multiple parents for a Group. In the netCDF-4 data model,

Groups form a tree with no cycles, so each Group (except the

top-level unnamed Group) has a unique parent.

- HDF5 supports "references" which are like pointers to objects and

data regions within a file. The netCDF-4 data model omits

references.

- HDF5 supports some primitive types that are not included in the

netCDF-4 data model, including H5T_TIME and H5T_BITFIELD.

- HDF5 supports multiple names for data objects like Datasets

(netCDF-4 variables) with no distinguished name. The netCDF-4 data

model requires that each variable, attribute, dimension, and group

have a single distinguished name.

- HDF5 (like netCDF) supports scalar attributes, but netCDF-4 cannot

read scalar HDF5 attributes (unless it is a string

attribute). This limitation will be removed in a future release of

netCDF.

These are fairly easy requirements to meet, but there is one relating

to shared dimensions which is a little more challenging. Every HDF5

dataset must have a dimension scale attached to each dimension.

Dimension scales are a new feature for HF 1.8, which allow

specification of shared dimensions.

Without creation order in the HDF5 file, the files will still be

readable to netCDF-4, it's just that netCDF-4 will number the

variables in alphabetical, rather than creation, order.

Interoperability is a complex task, and all of this is in the alpha

release stage. It is tested in libsrc4/tst_interops.c, which contains

some examples of how to create HDF5 files, modify them in netCDF-4,

and then verify them in HDF5. (And vice versa).

\page dap_support DAP Support

Beginning with netCDF version 4.1, optional support is provided for

accessing data through OPeNDAP servers using the DAP protocol.

Currently, only DAP protocol version 2 is supported; DAP protocol

version 4 support is under development.

DAP support is automatically enabled if a usable curl library can be

set using the LDFLAGS environment variable (similar to the way

that the HDF5 libraries are referenced).

DAP support can forcibly be enabled or disabled using the --enable-dap

flag or the --disable-dap flag, respectively.  If enabled,

then DAP2 support requires access to the curl library.

Refer to the installation manual for details

DAP uses a data model that is different from that supported by netCDF,

either classic or enhanced. Generically, the DAP data model is encoded

textually in a DDS (Dataset Descriptor Structure). There is a second

data model for DAP attributes, which is encoded textually in a DAS

(Dataset Attribute Structure). For detailed information about the DAP

DDS and DAS, refer to the OPeNDAP web site http://opendap.org.

\section Accessing OPeNDAP Data

In order to access an OPeNDAP data source through the netCDF API, the

file name normally used is replaced with a URL with a specific

format. The URL is composed of three parts.

-  URL - this is a standard form URL such as

http://thredds-test.ucar.edu/dts/test.01

-  Constraints - these are suffixed to the URL and take the form

“?\<projections>&\<selections>”. The meaning of the terms projection

and selection is somewhat complicated; and the OPeNDAP web site,

http://www.opendap.org, should be consulted. The interaction of DAP

constraints with netCDF is complex and at the moment requires an

understanding of how DAP is translated to netCDF.

- Client parameters - these may be specified in either of

two ways.  The older, deprecated form prefixes text to the

front of the url and is of the the general form [\<name>]

or [\<name>=value].  Examples include [show=fetch] and

[noprefetch].  The newer, preferred form prefixes the

parameters to the end of the url using the semi-standard '#'

format: e.g. http://....#show=fetch&noprefetch.

It is possible to see what the translation does to a particular DAP

data source in either of two ways. First, one can examine the DDS

source through a web browser and then examine the translation using

the ncdump -h command to see the netCDF Classic translation. The

ncdump output will actually be the union of the DDS with the DAS, so

to see the complete translation, it is necessary to view both.

For example, if a web browser is given the following, the first URL

will return the DDS for the specified dataset, and the second URL will

return the DAS for the specified dataset.

\code

http://thredds-test.ucar.edu/dts/test.01.dds

http://thredds-test.ucar.edu/dts/test.01.das

\endcode

Then by using the following ncdump command, it is possible to see the

equivalent netCDF Classic translation.

\code

ncdump -h http://thredds-test.ucar.edu/dts/test.01

\endcode

The DDS output from the web server should look like this.

\code

Dataset {

Byte b;

Int32 i32;

UInt32 ui32;

Int16 i16;

UInt16 ui16;

Float32 f32;

Float64 f64;

String s;

Url u;

} SimpleTypes;

\endcode

The DAS output from the web server should look like this.

\code

Attributes {

Facility {

String PrincipleInvestigator ``Mark Abbott'', ``Ph.D'';

String DataCenter ``COAS Environmental Computer Facility'';

String DrifterType ``MetOcean WOCE/OCM'';

}

b {

String Description ``A test byte'';

String units ``unknown'';

}

i32 {

String Description ``A 32 bit test server int'';

String units ``unknown'';

}

}

\endcode

The output from ncdump should look like this.

\code

netcdf test {

dimensions:

stringdim64 = 64 ;

variables:

byte b ;

b:Description = "A test byte" ;

b:units = "unknown" ;

int i32 ;

i32:Description = "A 32 bit test server int" ;

i32:units = "unknown" ;

int ui32 ;

short i16 ;

short ui16 ;

float f32 ;

double f64 ;

char s(stringdim64) ;

char u(stringdim64) ;

}

\endcode

Note that the fields of type String and type URL have suddenly

acquired a dimension. This is because strings are translated to arrays

of char, which requires adding an extra dimension. The size of the

dimension is determined in a variety of ways and can be specified. It

defaults to 64 and when read, the underlying string is either padded

or truncated to that length.

Also note that the Facility attributes do not appear in the

translation because they are neither global nor associated with a

variable in the DDS.

Alternately, one can get the text of the DDS as a global attribute by

using the client parameters mechanism . In this case, the parameter

“show=dds” can be used, and the data retrieved using

the following command

\code

ncdump -h http://thredds-test.ucar.edu/dts/test.01.dds#show=dds

\endcode

The ncdump -h command will then show both the translation and the

original DDS. In the above example, the DDS would appear as the global

attribute “_DDS” as follows.

\code

netcdf test {

...

variables:

:_DDS = "Dataset { Byte b; Int32 i32; UInt32 ui32; Int16 i16;

UInt16 ui16; Float32 f32; Float64 f64;

Strings; Url u; } SimpleTypes;"

byte b ;

...

}

\endcode

\section dap_to_netcdf DAP to NetCDF Translation Rules

Currently only one translation available: DAP 2 Protocol to netCDF-3.

There used to be a DAP 2 Protocol to netCDF-4 translation

but that has been removed until the DAP4 protocol is available.

\subsection nc3_trans_rules netCDF-3 Translation Rules

The current default translation code translates the OPeNDAP protocol

to netCDF-3 (classic). This netCDF-3 translation converts an OPeNDAP

DAP protocol version 2 DDS to netCDF-3 and is designed to mimic as

closely as possible the translation provided by the libnc-dap

system, except that some errors in that older translation have

been fixed.

For illustrative purposes, the following example will be used.

\code

Dataset {

Int32 f1;

Structure {

Int32 f11;

Structure {

Int32 f1[3];

Int32 f2;

} FS2[2];

} S1;

Structure {

Grid {

Array:

Float32 temp[lat=2][lon=2];

Maps:

Int32 lat[lat=2];

Int32 lon[lon=2];

} G1;

} S2;

Grid {

Array:

Float32 G2[lat=2][lon=2];

Maps:

Int32 lat[2];

Int32 lon[2];

} G2;

Int32 lat[lat=2];

Int32 lon[lon=2];

} D1;

\code

\subsection var_def Variable Definition

The set of netCDF variables is derived from the fields with primitive

base types as they occur in Sequences, Grids, and Structures. The

field names are modified to be fully qualified initially. For the

above, the set of variables are as follows. The coordinate variables

within grids are left out in order to mimic the behavior of libnc-dap.

\code

f1

S1.f11

S1.FS2.f1

S1.FS2.f2

S2.G1.temp

S2.G2.G2

lat

lon

\endcode

\subsection var_dim_trans Variable Dimension Translation

A variable's rank is determined from three sources.

- The variable has the dimensions associated with the field it

represents (e.g. S1.FS2.f1[3] in the above example).

- The variable inherits the dimensions associated with any containing

structure that has a rank greater than zero. These dimensions precede

those of case 1. Thus, we have in our example, f1[2][3], where the

first dimension comes from the containing Structure FS2[2].

- The variable's set of dimensions are altered if any of its

containers is a DAP DDS Sequence. This is discussed more fully below.

If the type of the netCDF variable is char, then an extra string

dimension is added as the last dimension.

\subsection dim_trans Dimension translation

For dimensions, the rules are as follows.

Fields in dimensioned structures inherit the dimension of the

structure; thus the above list would have the following dimensioned

variables.

\code

S1.FS2.f1 -> S1.FS2.f1[2][3]

S1.FS2.f2 -> S1.FS2.f2[2]

S2.G1.temp -> S2.G1.temp[lat=2][lon=2]

S2.G1.lat -> S2.G1.lat[lat=2]

S2.G1.lon -> S2.G1.lon[lon=2]

S2.G2.G2 -> S2.G2.lon[lat=2][lon=2]

S2.G2.lat -> S2.G2.lat[lat=2]

S2.G2.lon -> S2.G2.lon[lon=2]

lat -> lat[lat=2]

lon -> lon[lon=2]

\endcode

Collect all of the dimension specifications from the DDS, both named

and anonymous (unnamed) For each unique anonymous dimension with value

NN create a netCDF dimension of the form "XX_<i>=NN", where XX is the

fully qualified name of the variable and i is the i'th (inherited)

dimension of the array where the anonymous dimension occurs. For our

example, this would create the following dimensions.

\code

S1.FS2.f1_0 = 2 ;

S1.FS2.f1_1 = 3 ;

S1.FS2.f2_0 = 2 ;

S2.G2.lat_0 = 2 ;

S2.G2.lon_0 = 2 ;

\endcode

If however, the anonymous dimension is the single dimension of a MAP

vector in a Grid then the dimension is given the same name as the map

vector This leads to the following.

\code

S2.G2.lat_0 -> S2.G2.lat

S2.G2.lon_0 -> S2.G2.lon

\endcode

For each unique named dimension "<name>=NN", create a netCDF dimension

of the form "<name>=NN", where name has the qualifications removed. If

this leads to duplicates (i.e. same name and same value), then the

duplicates are ignored. This produces the following.

\code

S2.G2.lat -> lat

S2.G2.lon -> lon

\endcode

Note that this produces duplicates that will be ignored later.

At this point the only dimensions left to process should be named

dimensions with the same name as some dimension from step number 3,

but with a different value. For those dimensions create a dimension of

the form "<name>M=NN" where M is a counter starting at 1. The example

has no instances of this.

Finally and if needed, define a single UNLIMITED dimension named

"unlimited" with value zero. Unlimited will be used to handle certain

kinds of DAP sequences (see below).

This leads to the following set of dimensions.

\code

dimensions:

unlimited = UNLIMITED;

lat = 2 ;

lon = 2 ;

S1.FS2.f1_0 = 2 ;

S1.FS2.f1_1 = 3 ;

S1.FS2.f2_0 = 2 ;

\endcode

\subsection var_name_trans Variable Name Translation

The steps for variable name translation are as follows.

Take the set of variables captured above. Thus for the above DDS, the

following fields would be collected.

\code

f1

S1.f11

S1.FS2.f1

S1.FS2.f2

S2.G1.temp

S2.G2.G2

lat

lon

\endcode

All grid array variables are renamed to be the same as the containing

grid and the grid prefix is removed. In the above DDS, this results in

the following changes.

\code

G1.temp -> G1

G2.G2 -> G2

\endcode

It is important to note that this process could produce duplicate

variables (i.e. with the same name); in that case they are all assumed

to have the same content and the duplicates are ignored. If it turns

out that the duplicates have different content, then the translation

will not detect this. YOU HAVE BEEN WARNED.

The final netCDF-3 schema (minus attributes) is then as follows.

\code

netcdf t {

dimensions:

unlimited = UNLIMITED ;

lat = 2 ;

lon = 2 ;

S1.FS2.f1_0 = 2 ;

S1.FS2.f1_1 = 3 ;

S1.FS2.f2_0 = 2 ;

variables:

int f1 ;

int lat(lat) ;

int lon(lon) ;

int S1.f11 ;

int S1.FS2.f1(S1.FS2.f1_0, S1.FS2.f1_1) ;

int S1.FS2.f2(S1_FS2_f2_0) ;

float S2.G1(lat, lon) ;

float G2(lat, lon) ;

}

\endcode

In actuality, the unlimited dimension is dropped because it is unused.

There are differences with the original libnc-dap here because

libnc-dap technically was incorrect. The original would have said

this, for example.

\code

int S1.FS2.f1(lat, lat) ;

\endcode

Note that this is incorrect because it dimensions S1.FS2.f1(2,2)

rather than S1.FS2.f1(2,3).

\subsection translation Translating DAP DDS Sequences

Any variable (as determined above) that is contained directly or

indirectly by a Sequence is subject to revision of its rank using the

following rules.

Let the variable be contained in Sequence Q1, where Q1 is the

innermost containing sequence. If Q1 is itself contained (directly or

indirectly) in a sequence, or Q1 is contained (again directly or

indirectly) in a structure that has rank greater than 0, then the

variable will have an initial UNLIMITED dimension. Further, all

dimensions coming from "above" and including (in the containment

sense) the innermost Sequence, Q1, will be removed and replaced by

that single UNLIMITED dimension. The size associated with that

UNLIMITED is zero, which means that its contents are inaccessible

through the netCDF-3 API. Again, this differs from libnc-dap, which

leaves out such variables. Again, however, this difference is backward

compatible.

If the variable is contained in a single Sequence (i.e. not nested)

and all containing structures have rank 0, then the variable will have

an initial dimension whose size is the record count for that

Sequence. The name of the new dimension will be the name of the

Sequence.

Consider this example.

\code

Dataset {

Structure {

Sequence {

Int32 f1[3];

Int32 f2;

} SQ1;

} S1[2];

Sequence {

Structure {

Int32 x1[7];

} S2[5];

} Q2;

} D;

\endcode

The corresponding netCDF-3 translation is pretty much as follows (the

value for dimension Q2 may differ).

\code

dimensions:

unlimited = UNLIMITED ; // (0 currently)

S1.SQ1.f1_0 = 2 ;

S1.SQ1.f1_1 = 3 ;

S1.SQ1.f2_0 = 2 ;

Q2.S2.x1_0 = 5 ;

Q2.S2.x1_1 = 7 ;

Q2 = 5 ;

variables:

int S1.SQ1.f1(unlimited, S1.SQ1.f1_1) ;

int S1.SQ1.f2(unlimited) ;

int Q2.S2.x1(Q2, Q2.S2.x1_0, Q2.S2.x1_1) ;

\endcode

Note that for example S1.SQ1.f1_0 is not actually used because it has

been folded into the unlimited dimension.

Note that for sequences without a leading unlimited dimension, there

is a performance cost because the translation code has to walk the

data to determine how many records are associated with the

sequence. Since libnc-dap did essentially the same thing, it can be

assumed that the cost is not prohibitive.

\subsection caching Caching

In an effort to provide better performance for some access patterns,

client-side caching of data is available. The default is no caching,

but it may be enabled by prefixing the URL with the paramter "cache".

Caching operates basically as follows.

When a URL is first accessed using nc_open(), netCDF automatically

does a pre-fetch of selected variables. These include all variables

smaller than a specified (and user definable) size. This allows, for

example, quick access to coordinate variables. This can be suppressed

with the parameter "noprefetch".

Whenever a request is made using some variant of the nc_get_var() API

procedures, the complete variable is fetched and stored in the cache

as a new cache entry. Subsequence requests for any part of that

variable will access the cache entry to obtain the data.

The cache may become too full, either because there are too many

entries or because it is taking up too much disk space. In this case

cache entries are purged until the cache size limits are reached. The

cache purge algorithm is LRU (least recently used) so that variables

that are repeatedly referenced will tend to stay in the cache.

The cache is completely purged when nc_close() is invoked.

In order to decide if you should enable caching, you will need to have

some understanding of the access patterns of your program.

The ncdump program always dumps one or more whole variables so it

turns on caching.

If your program accesses only parts of a number of variables, then

caching should probably not be used since fetching whole variables

will probably slow down your program for no purpose.

Unfortunately, caching is currently an all or nothing proposition, so

for more complex access patterns, the decision to cache or not may not

have an obvious answer. Probably a good rule of thumb is to avoid

caching initially and later turn it on to see its effect on

performance.

\subsection defined_params Defined Client Parameters

Currently, a limited set of client parameters is

recognized. Parameters not listed here are ignored, but no error is

signalled.

Parameter Name Legal Values Semantics

- "log" | "log=<file>" - Turn on logging and send the log output to

the specified file. If no file is specified, then output is sent to standard

error.

- "show=... das|dds|url" - This causes information to appear as

specific global attributes. The currently recognized tags are "dds"

to display the underlying DDS, "das" similarly, and "url" to display

the url used to retrieve the data. This parameter may be specified

multiple times (e.g. “show=dds&show=url”).

- "show=fetch" - This parameter causes the netCDF code to log a copy

of the complete url for every HTTP get request. If logging is

enabled, then this can be helpful in checking to see the access

behavior of the netCDF code.

- "stringlength=NN" - Specify the default string length to use for

string dimensions. The default is 64.

- "stringlength_<var>=NN" - Specify the default string length to use

for a string dimension for the specified variable. The default is

64.

- "cache" - This enables caching.

- "cachelimit=NN" - Specify the maximum amount of space allowed for

the cache.

- "cachecount=NN" - Specify the maximum number of entries in the

cache.

- "noprefetch" - This disables prefetch of small variables.

\subsection dap_debug Notes on Debugging OPeNDAP Access

The OPeNDAP support makes use of the logging facility of the

underlying oc system (see http://www.opendap.org/oc).

Note that this is currently separate from the

existing netCDF logging facility. Turning on this logging can

sometimes give important information. Logging can be enabled by

using the client parameter "log" or "log=filename",

where the first case will send log output to standard error and the

second will send log output to the specified file.

Users should also be aware that if one is

accessing data over an NFS mount, one may see some .nfsxxxxx files;

those can be ignored.

\subsection http_config HTTP Configuration.

Limited support for configuring the http connection is provided via

parameters in the “.dodsrc” configuration file. The relevant .dodsrc file is

located by first looking in the current working directory, and if not

found, then looking in the directory specified by the “$HOME”

environment variable.

Entries in the .dodsrc file are of the form:

\code

['['<url>']']<key>=<value>

\endcode

That is, it consists of a key name and value pair and optionally

preceded by a url enclosed in square brackets.

For given KEY and URL strings, the value chosen is as follows:

If URL is null, then look for the .dodsrc entry that has no url prefix

and whose key is same as the KEY for which we are looking.

If the URL is not null, then look for all the .dodsrc entries that

have a url, URL1, say, and for which URL1 is a prefix (in the string

sense) of URL. For example, if URL = http//x.y/a, then it will match

entries of the form

\code

1. [http//x.y/a]KEY=VALUE

2. [http//x.y/a/b]KEY=VALUE

\endcode

It will not match an entry of the form

\code

[http//x.y/b]KEY=VALUE

\endcode

because “http://x.y/b” is not a string prefix of

“http://x.y/a”. Finally from the set so constructed, choose the entry

with the longest url prefix: “http//x.y/a/b]KEY=VALUE” in this case.

Currently, the supported set of keys (with descriptions) are as

follows.

<pre>

HTTP.VERBOSE

Type: boolean ("1"/"0")

Description: Produce verbose output, especially using SSL.

Related CURL Flags: CURLOPT_VERBOSE

HTTP.DEFLATE

Type: boolean ("1"/"0")

Description: Allow use of compression by the server.

Related CURL Flags: CURLOPT_ENCODING

HTTP.COOKIEJAR

Type: String representing file path

Description: Specify the name of file into which to store cookies. Defaults to in-memory storage.

Related CURL Flags:CURLOPT_COOKIEJAR

HTTP.COOKIEFILE

Type: String representing file path

Description: Same as HTTP.COOKIEJAR.

Related CURL Flags: CURLOPT_COOKIEFILE

HTTP.CREDENTIALS.USER

Type: String representing user name

Description: Specify the user name for Digest and Basic authentication.

Related CURL Flags:

HTTP.CREDENTIALS.PASSWORD

Type: String representing password

Type: boolean ("1"/"0")

Description: Specify the password for Digest and Basic authentication.

Related CURL Flags:

HTTP.SSL.CERTIFICATE

Type: String representing file path

Description: Path to a file containing a PEM cerficate.

Related CURL Flags: CURLOPT_CERT

HTTP.SSL.KEY

Type: String representing file path

Description: Same as HTTP.SSL.CERTIFICATE, and should usually have the same value.

Related CURL Flags: CURLOPT_SSLKEY

HTTP.SSL.KEYPASSWORD

Type: String representing password

Description: Password for accessing the HTTP.SSL.KEY/HTTP.SSL.CERTIFICATE

Related CURL Flags: CURLOPT_KEYPASSWORD

HTTP.SSL.CAPATH

Type: String representing directory

Description: Path to a directory containing trusted certificates for validating server sertificates.

Related CURL Flags: CURLOPT_CAPATH

HTTP.SSL.VALIDATE

Type: boolean ("1"/"0")

Description: Cause the client to verify the server's presented certificate.

Related CURL Flags: CURLOPT_SSL_VERIFYPEER, CURLOPT_SSL_VERIFYHOST

HTTP.TIMEOUT

Type: String ("dddddd")

Description: Specify the maximum time in seconds that you allow the http transfer operation to take.

Related CURL Flags: CURLOPT_TIMEOUT, CURLOPT_NOSIGNAL

HTTP.PROXY_SERVER

Type: String representing url to access the proxy: (e.g.http://[username:password@]host[:port])

Description: Specify the needed information for accessing a proxy.

Related CURL Flags: CURLOPT_PROXY, CURLOPT_PROXYHOST, CURLOPT_PROXYUSERPWD

</pre>

The related curl flags line indicates the curl flags modified by this

key. See the libcurl documentation of the curl_easy_setopt() function

for more detail (http://curl.haxx.se/libcurl/c/curl_easy_setopt.html).

For ESG client side key support, the following entries must be specified:

\code

HTTP.SSL.VALIDATE

HTTP.COOKIEJAR

HTTP.SSL.CERTIFICATE

HTTP.SSL.KEY

HTTP.SSL.CAPATH

\endcode

Additionally, for ESG, the HTTP.SSL.CERTIFICATE and HTTP.SSL.KEY

entries should have same value, which is the file path for the

certificate produced by MyProxyLogon. The HTTP.SSL.CAPATH entry should

be the path to the "certificates" directory produced by MyProxyLogon.

\page chunk_cache The Chunk Cache

When data are first read or written to a netCDF-4/HDF5 variable, the

HDF5 library opens a cache for that variable. The default size of that

cache (settable with the –with-chunk-cache-size at netCDF build time).

For good performance your chunk cache must be larger than one chunk of

your data - preferably that it be large enough to hold multiple chunks

of data.

In addition, when a file is opened (or a variable created in an open

file), the netCDF-4 library checks to make sure the default chunk

cache size will work for that variable. The cache will be large enough

to hold N chunks, up to a maximum size of M bytes. (Both N and M are

settable at configure time with the –with-default-chunks-in-cache and

the –with-max-default-cache-size options to the configure

script. Currently they are set to 10 and 64 MB.)

To change the default chunk cache size, use the set_chunk_cache

function before opening the file with nc_set_chunk_cache(). Fortran 77

programmers see NF_SET_CHUNK_CACHE). Fortran 90 programmers use the

optional cache_size, cache_nelems, and cache_preemption parameters to

nf90_open/nf90_create to change the chunk size before opening the

file.

To change the per-variable cache size, use the set_var_chunk_cache

function at any time on an open file. C programmers see

nc_set_var_chunk_cache(), Fortran 77 programmers see

NF_SET_VAR_CHUNK_CACHE, ).

\page default_chunking_4_1 The Default Chunking Scheme

When the data writer does not specify chunk sizes for variable, the

netCDF library has to come up with some default values.

For unlimited dimensions, a chunk size of one is always used. For

large datasets, where the size of fixed dimensions is small compared

to the unlimited dimensions, users are advised to avoid unlimited

dimensions or to increase the chunk sizes of the unlimited

dimensions. Be aware that an unlimited dimension with chunksize > 1

may result in slower performance for record-oriented access patterns

that where common with netcdf-3.

For unlimited dimensions, a chunk size of one is always used. For

large datasets, where the size of fixed dimensions is small compared

to the unlimited dimensions, users are advised to avoid unlimited

dimensions or to increase the chunk sizes of the unlimited

dimensions. Be aware that an unlimited dimension with chunksize > 1

may result in slower performance for record-oriented access patterns

that where common with netcdf-3.

\page chunking_parallel_io Chunking and Parallel I/O

When files are opened for read/write parallel I/O access, the chunk

cache is not used. Therefore it is important to open parallel files

with read only access when possible, to achieve the best performance.

\page bm_file A Utility to Help Benchmark Results: bm_file

The bm_file utility may be used to copy files, from one netCDF format

to another, changing chunking, filter, parallel I/O, and other

parameters. This program may be used for benchmarking netCDF

performance for user data files with a range of choices, allowing data

producers to pick settings that best serve their user base.

NetCDF must have been configured with –enable-benchmarks at build time

for the bm_file program to be built. When built with

–enable-benchmarks, netCDF will include tests (run with “make check”)

that will run the bm_file program on sample data files.

Since data files and their access patterns vary from case to case,

these benchmark tests are intended to suggest further use of the

bm_file program for users.

Here's an example of a call to bm_file:

\code

./bm_file -d -f 3 -o  tst_elena_out.nc -c 0:-1:0:1024:256:256 tst_elena_int_3D.nc

\endcode

Generally a range of settings must be tested. This is best done with a

shell script, which calls bf_file repeatedly, to create output like

this:

<pre>

*** Running benchmarking program bm_file for simple shorts test files, 1D to 6D...

input format, output_format, input size, output size, meta read time, meta write time, data read time, data write time, enddianness, metadata reread time, data reread time, read rate, write rate, reread rate, deflate, shuffle, chunksize[0], chunksize[1], chunksize[2], chunksize[3]

1, 4, 200092, 207283, 1613, 1054, 409, 312, 0, 1208, 1551, 488.998, 641.026, 128.949, 0, 0, 100000, 0, 0, 0

1, 4, 199824, 208093, 1545, 1293, 397, 284, 0, 1382, 1563, 503.053, 703.211, 127.775, 0, 0, 316, 316, 0, 0

1, 4, 194804, 204260, 1562, 1611, 390, 10704, 0, 1627, 2578, 499.159, 18.1868, 75.5128, 0, 0, 46, 46, 46, 0

1, 4, 167196, 177744, 1531, 1888, 330, 265, 0, 12888, 1301, 506.188, 630.347, 128.395, 0, 0, 17, 17, 17, 17

1, 4, 200172, 211821, 1509, 2065, 422, 308, 0, 1979, 1550, 473.934, 649.351, 129.032, 0, 0, 10, 10, 10, 10

1, 4, 93504, 106272, 1496, 2467, 191, 214, 0, 32208, 809, 488.544, 436.037, 115.342, 0, 0, 6, 6, 6, 6

*** SUCCESS!!!

</pre>

Such tables are suitable for import into spreadsheets, for easy

graphing of results.

Several test scripts are run during the “make check” of the netCDF

build, in the nc_test4 directory. The following example may be found

in nc_test4/run_bm_elena.sh.

<pre>

#!/bin/sh

# This shell runs some benchmarks that Elena ran as described here:

# http://hdfeos.org/workshops/ws06/presentations/Pourmal/HDF5_IO_Perf.pdf

# $Id: netcdf.texi,v 1.82 2010/05/15 20:43:13 dmh Exp $

set -e

echo ""

echo "*** Testing the benchmarking program bm_file for simple float file, no compression..."

./bm_file -h -d -f 3 -o  tst_elena_out.nc -c 0:-1:0:1024:16:256 tst_elena_int_3D.nc

./bm_file -d -f 3 -o  tst_elena_out.nc -c 0:-1:0:1024:256:256 tst_elena_int_3D.nc

./bm_file -d -f 3 -o  tst_elena_out.nc -c 0:-1:0:512:64:256 tst_elena_int_3D.nc

./bm_file -d -f 3 -o  tst_elena_out.nc -c 0:-1:0:512:256:256 tst_elena_int_3D.nc

./bm_file -d -f 3 -o  tst_elena_out.nc -c 0:-1:0:256:64:256 tst_elena_int_3D.nc

./bm_file -d -f 3 -o  tst_elena_out.nc -c 0:-1:0:256:256:256 tst_elena_int_3D.nc

echo '*** SUCCESS!!!'

exit 0

</pre>

The reading that bm_file does can be tailored to match the expected

access pattern.

The bm_file program is controlled with command line options.

<pre>

./bm_file

bm_file -v [-s N]|[-t V:S:S:S -u V:C:C:C -r V:I:I:I] -o file_out -f N -h -c V:C:C,V:C:C:C -d -m -p -i -e 1|2 file

[-v]        Verbose

[-o file]   Output file name

[-f N]      Output format (1 - classic, 2 - 64-bit offset, 3 - netCDF-4, 4 - netCDF4/CLASSIC)

[-h]        Print output header

[-c V:Z:S:C:C:C[,V:Z:S:C:C:C, etc.]] Deflate, shuffle, and chunking parameters for vars

[-t V:S:S:S[,V:S:S:S, etc.]] Starts for reads/writes

[-u V:C:C:C[,V:C:C:C, etc.]] Counts for reads/writes

[-r V:I:I:I[,V:I:I:I, etc.]] Incs for reads/writes

[-d]        Doublecheck output by rereading each value

[-m]        Do compare of each data value during doublecheck (slow for large files!)

[-p]        Use parallel I/O

[-s N]      Denom of fraction of slowest varying dimension read.

[-i]        Use MPIIO (only relevant for parallel builds).

[-e 1|2]    Set the endianness of output (1=little 2=big).

file        Name of netCDF file

</pre>

\page cdl_syntax CDL Syntax

Below is an example of CDL, describing a netCDF classic format file with several

named dimensions (lat, lon, time), variables (z, t, p, rh, lat, lon,

time), variable attributes (units, _FillValue, valid_range), and some

data.

\code

netcdf foo {    // example netCDF specification in CDL

dimensions:

lat = 10, lon = 5, time = unlimited;

variables:

int     lat(lat), lon(lon), time(time);

float   z(time,lat,lon), t(time,lat,lon);

double  p(time,lat,lon);

int     rh(time,lat,lon);

lat:units = "degrees_north";

lon:units = "degrees_east";

time:units = "seconds";

z:units = "meters";

z:valid_range = 0., 5000.;

p:_FillValue = -9999.;

rh:_FillValue = -1;

data:

lat   = 0, 10, 20, 30, 40, 50, 60, 70, 80, 90;

lon   = -140, -118, -96, -84, -52;

}

\endcode

All CDL statements are terminated by a semicolon. Spaces, tabs, and

newlines can be used freely for readability. Comments may follow the

double slash characters '//' on any line.

A CDL description for a classic model file consists of three optional

parts: dimensions, variables, and data. The variable part may contain

variable declarations and attribute assignments. For the enhanced

model supported by netCDF-4, a CDL decription may also includes

groups, subgroups, and user-defined types.

A dimension is used to define the shape of one or more of the

multidimensional variables described by the CDL description. A

dimension has a name and a length. At most one dimension in a classic

CDL description can have the unlimited length, which means a variable

using this dimension can grow to any length (like a record number in a

file). Any number of dimensions can be declared of unlimited length in

CDL for an enhanced model file.

A variable represents a multidimensional array of values of the same

type. A variable has a name, a data type, and a shape described by its

list of dimensions. Each variable may also have associated attributes

(see below) as well as data values. The name, data type, and shape of

a variable are specified by its declaration in the variable section of

a CDL description. A variable may have the same name as a dimension;

by convention such a variable contains coordinates of the dimension it

names.

An attribute contains information about a variable or about the whole

netCDF dataset or containing group. Attributes may be used to specify

such properties as units, special values, maximum and minimum valid

values, and packing parameters. Attribute information is represented

by single values or one-dimensional arrays of values. For example,

“units” might be an attribute represented by a string such as

“celsius”. An attribute has an associated variable, a name, a data

type, a length, and a value. In contrast to variables that are

intended for data, attributes are intended for ancillary data or

metadata (data about data).

In CDL, an attribute is designated by a variable and attribute name,

separated by a colon (':'). It is possible to assign global attributes

to the netCDF dataset as a whole by omitting the variable name and

beginning the attribute name with a colon (':'). The data type of an

attribute in CDL, if not explicitly specified, is derived from the

type of the value assigned to it. In the netCDF-4

enhanced model, attributes may be declared to be of user-defined type,

like variables.

The length of an attribute is the number of data values assigned to

it.  Multiple values are assigned to non-character attributes by

separating the values with commas (',').  All values assigned to an

attribute must be of the same type.  In the classic data model,

character arrays are used for textual information.  The length of a

character attribute is the number of bytes, and an array of

character values can be represented in string notation.  In the

enhanced data model of netCDF-4, variable-length strings are available

as a primitive type, and the length of a string attribute is the

number of string values assigned to it.

In CDL, just as for netCDF, the names of dimensions, variables and

attributes (and, in netCDF-4 files, groups, user-defined types,

compound member names, and enumeration symbols) consist of arbitrary

sequences of alphanumeric characters, underscore '_', period '.', plus

'+', hyphen '-', or at sign '@', but beginning with a letter or

underscore. However names commencing with underscore are reserved for

system use. Case is significant in netCDF names. A zero-length name is

not allowed. Some widely used conventions restrict names to only

alphanumeric characters or underscores. Names that have trailing space

characters are also not permitted.

Beginning with versions 3.6.3 and 4.0, names may also include UTF-8

encoded Unicode characters as well as other special characters, except

for the character '/', which may not appear in a name (because it is

reserved for path names of nested groups). In CDL, most special

characters are escaped with a backslash '\' character, but that

character is not actually part of the netCDF name. The special

characters that do not need to be escaped in CDL names are underscore

'_', period '.', plus '+', hyphen '-', or at sign '@'.  The formal

specification of CDL name syntax is provided in the classic format

specification (see \ref classic_format).  Note that by using

special characters in names, you may make your data not compliant with

conventions that have more stringent requirements on valid names for

netCDF components, for example the CF Conventions.

The names for the primitive data types are reserved words in CDL, so

names of variables, dimensions, and attributes must not be primitive

type names.

The optional data section of a CDL description is where netCDF

variables may be initialized. The syntax of an initialization is

simple:

\code

variable = value_1, value_2, ... ;

\endcode

The comma-delimited list of constants may be separated by spaces,

tabs, and newlines. For multidimensional arrays, the last dimension

varies fastest. Thus, row-order rather than column order is used for

matrices. If fewer values are supplied than are needed to fill a

variable, it is extended with the fill value. The types of constants

need not match the type declared for a variable; coercions are done to

convert integers to floating point, for example. All meaningful type

conversions among numeric primitive types are supported.

A special notation for fill values is supported: the ‘_’ character

designates a fill value for variables.

\page cdl_data_types CDL Data Types

The CDL primitive data types for the classic model are:

- char - Characters.

- byte - Eight-bit integers.

- short - 16-bit signed integers.

- int - 32-bit signed integers.

- long - (Deprecated, synonymous with int)

- float - IEEE single-precision floating point (32 bits).

- real - (Synonymous with float).

- double - IEEE double-precision floating point (64 bits).

NetCDF-4 supports the additional primitive types:

- ubyte - Unsigned eight-bit integers.

- ushort - Unsigned 16-bit integers.

- uint - Unsigned 32-bit integers.

- int64 - 64-bit singed integers.

- uint64 - Unsigned 64-bit singed integers.

- string - Variable-length string of characters

Except for the added numeric data-types byte and ubyte, CDL supports

the same numeric primitive

data types as C. For backward compatibility, in declarations primitive

type names may be specified in either upper or lower case.

The byte type differs from the char type in that it is intended for

numeric data, and the zero byte has no special significance, as it may

for character data. In the classic data model, byte data could be

interpreted as either signed (-128 to 127) or unsigned (0 to

255). When reading byte data in a way that converts it into another

numeric type, the default interpretation is signed.  The netCDF-4

enhanced data model added an unsigned byte type.

The short type holds values between -32768 and

32767. The ushort type holds values between 0 and 65536. The int type

can hold values between -2147483648 and 2147483647. The uint type

holds values between 0 and 4294967296. The int64 type can hold values

between -9223372036854775808 and 9223372036854775807. The uint64 type

can hold values between 0 and 18446744073709551616.

The float type can hold values between about -3.4+38 and 3.4+38, with

external representation as 32-bit IEEE normalized single-precision

floating-point numbers. The double type can hold values between about

-1.7+308 and 1.7+308, with external representation as 64-bit IEEE

standard normalized double-precision, floating-point numbers. The

string type holds variable length strings.

A netCDF-4 string is a variable length array of Unicode

<http://unicode.org/> characters. When reading/writing a String to a

netCDF file or other external representation, the characters are UTF-8

encoded <http://en.wikipedia.org/wiki/UTF-8> (note that ASCII is a

subset of UTF-8). Libraries may use different internal

representations, for example the Java library uses UTF-16 encoding.

The netCDF char type contains uninterpreted characters, one character

per byte.  Typically these contain 7-bit ASCII characters, but the

character encoding is application specific. For this reason,

applications writing data using the enhanced data model are encouraged

to use the netCDF-4 string data type in preference to the char data

type.  Applications writing string data using the char data type are

encouraged to add the special variable attribute "_Encoding" with a

value that the netCDF libraries recognize. Currently those valid

values are "UTF-8" or "ASCII", case insensitive.

\page cdl_constants CDL Notation for Data Constants

This section describes the CDL notation for constants.

Attributes are initialized in the variables section of a CDL

description by providing a list of constants that determines the

attribute's length and type (if primitive and not explicitly

declared). CDL defines a syntax for constant values that permits

distinguishing among different netCDF primitive types. The syntax for

CDL constants is similar to C syntax, with type suffixes appended to

bytes, shorts, and floats to distinguish them from ints and doubles.

A byte constant is represented by an integer constant with a 'b' (or

'B') appended.  In the old netCDF-2 API, byte constants could also be

represented using single characters or standard C character escape

sequences such as 'a' or '\n'.  This is still supported for backward

compatibility, but deprecated to make the distinction clear between

the numeric byte type and the textual char type.  Example byte

constants include:

\code

0b      // a zero byte

-1b     // -1 as an 8-bit byte

255b    // also -1 as a signed 8-bit byte

\endcode

Character constants are enclosed in double quotes. A character array

may be represented as a string enclosed in double quotes. Multiple CDL

strings are concatenated into a single array of characters, permitting

long character arrays to appear on multiple lines. To support multiple

variable-length textual values, a conventional delimiter such as ','

or blank may be used, but interpretation of any such convention for a

delimiter must be implemented in software above the netCDF library

layer. The usual escape conventions for C strings are honored. For

example:

\code

"a"            // ASCII 'a'

"Two\nlines\n" // a 10-character string with two embedded newlines

"a bell:\007"  // a character array containing an ASCII bell

"ab","cde"     // the same as "abcde"

\endcode

The form of a short constant is an integer constant with an 's' or 'S'

appended. If a short constant begins with '0', it is interpreted as

octal. When it begins with '0x', it is interpreted as a hexadecimal

constant. For example:

\code

2s      // a short 2

0123s   // octal

0x7ffs  // hexadecimal

\endcode

The form of an int constant is an ordinary integer constant. If an int

constant begins with '0', it is interpreted as octal. When it begins

with '0x', it is interpreted as a hexadecimal constant. Examples of

valid int constants include:

\code

-2

0123            // octal

0x7ff           // hexadecimal

1234567890L     // deprecated, uses old long suffix

\endcode

The float type is appropriate for representing data with about seven

significant digits of precision. The form of a float constant is the

same as a C floating-point constant with an 'f' or 'F' appended. A

decimal point is required in a CDL float to distinguish it from an

integer. For example, the following are all acceptable float

constants:

\code

-2.0f

3.14159265358979f       // will be truncated to less precision

1.f

.1f

\endcode

The double type is appropriate for representing floating-point data

with about 16 significant digits of precision. The form of a double

constant is the same as a C floating-point constant. An optional 'd'

or 'D' may be appended. A decimal point is required in a CDL double to

distinguish it from an integer. For example, the following are all

acceptable double constants:

\code

-2.0

3.141592653589793

1.0e-20

1.d

\endcode

Unsigned integer constants can be created by appending the character

'U' or 'u' between the constant and any trailing size specifier.  Thus

one could say 10U, 100us, 100000ul, or 1000000ull, for example.

Constants for the variable-length string type, available as a

primitive type in the netCDF-4 enhanced data model are, like character

constants, represented using double quotes. This represents a

potential ambiguity since a multi-character string may also indicate a

dimensioned character value. Disambiguation usually occurs by context,

but care should be taken to specify the string type to ensure the

proper choice.  For example, these two CDL specifications of global

attributes have different types:

\code

:att1 = "abcd", "efg" ;       // a char attribute of length 7

string :att2 = "abcd", efg" ; // a string attribute of length 2

\endcode

Opaque constants are represented as sequences of hexadecimal digits

preceded by 0X or 0x: 0xaa34ffff, for example.  These constants can

still be used as integer constants and will be either truncated or

extended as necessary.

The ncgen man-page reference has more details about CDL representation

of constants of user-defined types.

\page guide_ncgen ncgen

The ncgen tool generates a netCDF file or a C or FORTRAN program that

creates a netCDF dataset. If no options are specified in invoking

ncgen, the program merely checks the syntax of the CDL input,

producing error messages for any violations of CDL syntax.

The ncgen tool is now is capable of producing netcdf-4 files. It

operates essentially identically to the original ncgen.

The CDL input to ncgen may include data model constructs from the

netcdf- data model. In particular, it includes new primitive types

such as unsigned integers and strings, opaque data, enumerations, and

user-defined constructs using vlen and compound types. The ncgen man

page should be consulted for more detailed information.

UNIX syntax for invoking ncgen:

\code

ncgen [-b] [-o netcdf-file] [-c] [-f] [-k<kind>] [-l<language>] [-x] [input-file]

\endcode

where:

<pre>

-b

Create a (binary) netCDF file. If the '-o' option is absent, a default

file name will be constructed from the netCDF name (specified after

the netcdf keyword in the input) by appending the '.nc'

extension. Warning: if a file already exists with the specified name

it will be overwritten.

-o netcdf-file

Name for the netCDF file created. If this option is specified, it

implies the '-b' option. (This option is necessary because netCDF

files are direct-access files created with seek calls, and hence

cannot be written to standard output.)

-c

Generate C source code that will create a netCDF dataset matching the

netCDF specification. The C source code is written to standard

output. This is only useful for relatively small CDL files, since all

the data is included in variable initializations in the generated

program. The -c flag is deprecated and the -lc flag should be used

intstead.

-f

Generate FORTRAN source code that will create a netCDF dataset

matching the netCDF specification. The FORTRAN source code is written

to standard output. This is only useful for relatively small CDL

files, since all the data is included in variable initializations in

the generated program. The -f flag is deprecated and the -lf77 flag

should be used intstead.

-k

The -k file specifies the kind of netCDF file to generate. The

arguments to the -k flag can be as follows.

1, classic – Produce a netcdf classic file format file.

2, 64-bit-offset, '64-bit offset' – Produce a netcdf 64 bit classic file format file.

3, hdf5, netCDF-4, enhanced – Produce a netcdf-4 format file.

4, hdf5-nc3, 'netCDF-4 classic model', enhanced-nc3 – Produce a netcdf-4 file format, but restricted to netcdf-3 classic CDL input.

Note that the -v flag is a deprecated alias for -k.

-l

The -l file specifies that ncgen should output (to standard output)

the text of a program that, when compiled and executed, will produce

the corresponding binary .nc file. The arguments to the -l flag can be

as follows.

c|C => C language output.

f77|fortran77 => FORTRAN 77 language output; note that currently only the classic model is supported for fortran output.

-x

Use “no fill” mode, omitting the initialization of variable values

with fill values. This can make the creation of large files much

faster, but it will also eliminate the possibility of detecting the

inadvertent reading of values that haven't been written.

</pre>

<h1>Examples</h1>

Check the syntax of the CDL file foo.cdl:

\code

ncgen foo.cdl

\endcode

From the CDL file foo.cdl, generate an equivalent binary netCDF file

named bar.nc:

\code

ncgen -o bar.nc foo.cdl

\endcode

From the CDL file foo.cdl, generate a C program containing netCDF

function invocations that will create an equivalent binary netCDF

dataset:

\code

ncgen -l c foo.cdl > foo.c

\endcode

\page guide_ncdump ncdump

The \b ncdump utility generates a text representation of a specified

netCDF file on standard output, optionally excluding some or all of

the variable data in the output.  The text representation is in a form

called CDL (network Common Data form Language) that can be viewed,

edited, or serve as input to \b ncgen, a companion program that can

generate a binary netCDF file from a CDL file.  Hence \b ncgen and \b

ncdump can be used as inverses to transform the data representation

between binary and text representations.  See \b ncgen documentation

for a description of CDL and netCDF representations.

\b ncdump may also be used to determine what kind of netCDF file

is used (which variant of the netCDF file format) with the -k

option.

If DAP support was enabled when \b ncdump was built, the file name may

specify a DAP URL. This allows \b ncdump to access data sources from

DAP servers, including data in other formats than netCDF.  When used

with DAP URLs, \b ncdump shows the translation from the DAP data

model to the netCDF data model.

\b ncdump may also be used as a simple browser for netCDF data files,

to display the dimension names and lengths; variable names, types, and

shapes; attribute names and values; and optionally, the values of data

for all variables or selected variables in a netCDF file.  For

netCDF-4 files, groups and user-defined types are also included in \b

ncdump output.

\b ncdump uses '_' to represent data values that are equal to the

'_FillValue' attribute for a variable, intended to represent

data that has not yet been written.  If a variable has no

'_FillValue' attribute, the default fill value for the variable

type is used unless the variable is of byte type.

\b ncdump defines a default display format used for each type of

netCDF data, but this can be changed if a `C_format' attribute

is defined for a netCDF variable.  In this case, \b ncdump will

use the `C_format' attribute to format each value.  For

example, if floating-point data for the netCDF variable `Z' is

known to be accurate to only three significant digits, it would

be appropriate to use the variable attribute

\code

Z:C_format = "%.3g"

\endcode

@section  OPTIONS

@par -c

Show the values of \e coordinate \e variables (1D variables with the

same names as dimensions) as well as the declarations of all

dimensions, variables, attribute values, groups, and user-defined

types.  Data values of non-coordinate variables are not included in

the output.  This is usually the most suitable option to use for a

brief look at the structure and contents of a netCDF file.

@par -h

Show only the header information in the output, that is, output only

the declarations for the netCDF dimensions, variables, attributes,

groups, and user-defined types of the input file, but no data values

for any variables. The output is identical to using the '-c' option

except that the values of coordinate variables are not included. (At

most one of '-c' or '-h' options may be present.)

@par -v \a var1,...

@par

The output will include data values for the specified variables, in

addition to the declarations of all dimensions, variables, and

attributes. One or more variables must be specified by name in the

comma-delimited list following this option. The list must be a single

argument to the command, hence cannot contain unescaped blanks or

other white space characters. The named variables must be valid netCDF

variables in the input-file. A variable within a group in a netCDF-4

file may be specified with an absolute path name, such as

`/GroupA/GroupA2/var'.  Use of a relative path name such as `var' or

`grp/var' specifies all matching variable names in the file.  The

default, without this option and in the absence of the '-c' or '-h'

options, is to include data values for \e all variables in the output.

@par -b [c|f]

A brief annotation in the form of a CDL comment (text beginning with

the characters '//') will be included in the data section of the

output for each 'row' of data, to help identify data values for

multidimensional variables. If lang begins with 'C' or 'c', then C

language conventions will be used (zero-based indices, last dimension

varying fastest). If lang begins with 'F' or 'f', then FORTRAN

language conventions will be used (one-based indices, first dimension

varying fastest). In either case, the data will be presented in the

same order; only the annotations will differ. This option may be

useful for browsing through large volumes of multidimensional data.

@par -f [c|f]

Full annotations in the form of trailing CDL comments (text beginning

with the characters '//') for every data value (except individual

characters in character arrays) will be included in the data

section. If lang begins with 'C' or 'c', then C language conventions

will be used. If lang begins with 'F' or 'f', then FORTRAN language

conventions will be used. In either case, the data will be presented

in the same order; only the annotations will differ. This option may

be useful for piping data into other filters, since each data value

appears on a separate line, fully identified. (At most one of '-b' or

'-f' options may be present.)

@par -l \e length

@par

Changes the default maximum line length (80) used in formatting lists

of non-character data values.

@par -n \e name

@par

CDL requires a name for a netCDF file, for use by 'ncgen -b' in

generating a default netCDF file name. By default, \b ncdump

constructs this name from the last component of the file name of

the input netCDF file by stripping off any extension it has. Use

the '-n' option to specify a different name. Although the output

file name used by 'ncgen -b' can be specified, it may be wise to

have \b ncdump change the default name to avoid inadvertently

overwriting a valuable netCDF file when using \b ncdump, editing the

resulting CDL file, and using 'ncgen -b' to generate a new netCDF

file from the edited CDL file.

@par -p \e float_digits[, \e double_digits ]

@par

Specifies default precision (number of significant digits) to use in

displaying floating-point or double precision data values for

attributes and variables. If specified, this value overrides the value

of the C_format attribute, if any, for a variable. Floating-point data

will be displayed with \e float_digits significant digits. If \e

double_digits is also specified, double-precision values will be

displayed with that many significant digits. In the absence of any

'-p' specifications, floating-point and double-precision data are

displayed with 7 and 15 significant digits respectively. CDL files can

be made smaller if less precision is required. If both floating-point

and double precisions are specified, the two values must appear

separated by a comma (no blanks) as a single argument to the command.

(To represent every last bit of precision in a CDL file for all

possible floating-point values would require '-p 9,17'.)

@par -k

Show \e kind of netCDF file, that is which format variant the file uses.

Other options are ignored if this option is specified.  Output will be

one of 'classic'. '64-bit offset', 'netCDF-4', or 'netCDF-4 classic

model'.

@par -s

Specifies that \e special virtual attributes should be output for the

file format variant and for variable properties such as

compression, chunking, and other properties specific to the format

implementation that are primarily related to performance rather

than the logical schema of the data. All the special virtual

attributes begin with '_' followed by an upper-case

letter. Currently they include the global attribute '_Format' and

the variable attributes '_ChunkSizes', '_DeflateLevel',

'_Endianness', '_Fletcher32', '_NoFill', '_Shuffle', and '_Storage'.

The \b ncgen utility recognizes these attributes and

supports them appropriately.

@par -t

Controls display of time data, if stored in a variable that uses a

udunits compliant time representation such as 'days since 1970-01-01'

or 'seconds since 2009-03-15 12:01:17'.  If this option is specified,

time values are displayed as a human-readable date-time strings rather

than numerical values, interpreted in terms of a 'calendar' variable

attribute, if specified.  For numeric attributes of time variables,

the human-readable time value is displayed after the actual value, in

an associated CDL comment.  Calendar attribute values interpreted with

this option include the CF Conventions values 'gregorian' or

'standard', 'proleptic_gregorian', 'noleap' or '365_day', 'all_leap'

or '366_day', '360_day', and 'julian'.

@par -i

Same as the '-t' option, except output time data as date-time strings

with ISO-8601 standard 'T' separator, instead of a blank.

@par -g \e grp1,...

@par

The output will include data values only for the specified groups.

One or more groups must be specified by name in the comma-delimited

list following this option. The list must be a single argument to the

command. The named groups must be valid netCDF groups in the

input-file. The default, without this option and in the absence of the

'-c' or '-h' options, is to include data values for all groups in the

output.

@par -w

For file names that request remote access using DAP URLs, access data

with client-side caching of entire variables.

@par -x

Output XML (NcML) instead of CDL.  The NcML does not include data values.

The NcML output option currently only works for netCDF classic model data.

@section  EXAMPLES

Look at the structure of the data in the netCDF file foo.nc:

\code

ncdump -c foo.nc

\endcode

Produce an annotated CDL version of the structure and data in the

netCDF file foo.nc, using C-style indexing for the annotations:

\code

ncdump -b c foo.nc > foo.cdl

\endcode

Output data for only the variables uwind and vwind from the netCDF

file foo.nc, and show the floating-point data with only three

significant digits of precision:

\code

ncdump -v uwind,vwind -p 3 foo.nc

\endcode

Produce a fully-annotated (one data value per line) listing of the

data for the variable omega, using FORTRAN conventions for indices,

and changing the netCDF file name in the resulting CDL file to

omega:

\code

ncdump -v omega -f fortran -n omega foo.nc > Z.cdl

\endcode

Examine the translated DDS for the DAP source from the specified URL:

\code

ncdump -h http://thredds-test.ucar.edu/dts/test.01

\endcode

Without dumping all the data, show the special virtual attributes that indicate

performance-related characterisitics of a netCDF-4 file:

\code

ncdump -h -s nc4file.nc

\endcode

@section see_also SEE ALSO

ncgen(1), netcdf(3)

@section string_note NOTE ON STRING OUTPUT

For classic, 64-bit offset or netCDF-4 classic model data, \b ncdump

generates line breaks after embedded newlines in displaying character

data.  This is not done for netCDF-4 files, because netCDF-4 supports

arrays of real strings of varying length.

\page guide_nccopy nccopy

The \b nccopy utility copies an input netCDF file in any supported

format variant to an output netCDF file, optionally converting the

output to any compatible netCDF format variant, compressing the data,

or rechunking the data.  For example, if built with the netCDF-3

library, a netCDF classic file may be copied to a netCDF 64-bit offset

file, permitting larger variables.  If built with the netCDF-4

library, a netCDF classic file may be copied to a netCDF-4 file or to

a netCDF-4 classic model file as well, permitting data compression,

efficient schema changes, larger variable sizes, and use of other

netCDF-4 features.

\b nccopy also serves as an example of a generic netCDF-4 program,

with its ability to read any valid netCDF file and handle nested

groups, strings, and user-defined types, including arbitrarily

nested compound types, variable-length types, and data of any valid

netCDF-4 type.

If DAP support was enabled when \b nccopy was built, the file name may

specify a DAP URL. This may be used to convert data on DAP servers to

local netCDF files.

UNIX syntax for invoking nccopy:

\code

nccopy [-k kind] [-d n] [-s] [-u] [-w] [-c chunkspec] [-m bufsize]

[-h chunk_cache] [-e cache_elems] [-r]   infile   outfile

\endcode

where:

@par -k \e kind

Specifies the kind of file to be created (that is, the format variant)

and, by inference, the data model (i.e. netcdf-3 (classic) versus

netcdf-4 (enhanced)).  The possible arguments are as follows. \n

'1' or 'classic' => netCDF classic format \n

'2', '64-bit-offset', or '64-bit offset' => netCDF 64-bit format \n

'3', 'hdf5', 'netCDF-4', or 'enhanced' => netCDF-4 format (enhanced data model) \n

'4', 'hdf5-nc3', 'netCDF-4 classic model', or 'enhanced-nc3' => netCDF-4 classic model format \n

@par

If no value for -k is specified, then the output will use the same

format as the input, except if the input is classic or 64-bit offset

and either chunking or compression is specified, in which case the

output will be netCDF-4 classic model format.  Note that attempting

some kinds of format conversion will result in an error, if the

conversion is not possible.  For example, an attempt to copy a

netCDF-4 file that uses features of the enhanced model, such as groups

or variable-length strings, to any of the other kinds of netCDF

formats that use the classic model will result in an error.

@par -d \e n

For netCDF-4 output, including netCDF-4 classic model, specify

deflation level (level of compression) for variable data output.  0

corresponds to no compression and 9 to maximum compression, with

higher levels of compression requiring marginally more time to

compress or uncompress than lower levels.  Compression achieved may

also depend on output chunking parameters.  If this option is

specified for a classic format or 64-bit offset format input file, it

is not necessary to also specify that the output should be netCDF-4

classic model, as that will be the default.  If this option is not

specified and the input file has compressed variables, the compression

will still be preserved in the output, using the same chunking as in

the input by default.

@par

Note that \b nccopy requires all variables to be compressed using the

same compression level, but the API has no such restriction.  With a

program you can customize compression for each variable independently.

@par -s

For netCDF-4 output, including netCDF-4 classic model,

specify shuffling of variable data bytes before compression or after

decompression.  This option is ignored unless a non-zero deflation

level is specified.  Turning shuffling on sometimes improves

compression.

@par -u

Convert any unlimited size dimensions in the input to fixed size

dimensions in the output.  This can speed up variable-at-a-time

access, but slow down record-at-a-time access to multiple variables

along an unlimited dimension.

@par -w

Keep output in memory (as a diskless netCDF file) until output is

closed, at which time output file is written to disk.  This can

greatly speedup operations such as converting unlimited dimension to

fixed size (-u option), chunking, rechunking, or compressing the

input.  It requires that available memory is large enough to hold the

output file.  This option may provide a larger speedup than careful

tuning of the -m, -h, or -e options, and it's certainly a lot simpler.

@par -c  \e  chunkspec

@par

For netCDF-4 output, including netCDF-4 classic model, specify

chunking (multidimensional tiling) for variable data in the output.

This is useful to specify the units of disk access, compression, or

other filters such as checksums.  Changing the chunking in a netCDF

file can also greatly speedup access, by choosing chunk shapes that

are appropriate for the most common access patterns.

@par

The chunkspec argument is a string of comma-separated associations,

each specifying a dimension name, a '/' character, and optionally the

corresponding chunk length for that dimension.  No blanks should

appear in the chunkspec string, except possibly escaped blanks that

are part of a dimension name.  A chunkspec must name at least one

dimension, and may omit dimensions which are not to be chunked or for

which the default chunk length is desired.  If a dimension name is

followed by a '/' character but no subsequent chunk length, the actual

dimension length is assumed.  If copying a classic model file to a

netCDF-4 output file and not naming all dimensions in the chunkspec,

unnamed dimensions will also use the actual dimension length for the

chunk length.  An example of a chunkspec for variables that use

'm' and 'n' dimensions might be 'm/100,n/200' to specify 100 by 200

chunks.  To see the chunking resulting from copying with a chunkspec,

use the '-s' option of ncdump on the output file.

@par

Note that \b nccopy requires variables that share a dimension to also

share the chunk size associated with that dimension, but the

programming interface has no such restriction.  If you need to

customize chunking for variables independently, you will need to use

the library API in a custom utility program.

@par -m  \e  bufsize

@par

An integer or floating-point number that specifies the size, in bytes,

of the copy buffer used to copy large variables.  A suffix of K, M, G,

or T multiplies the copy buffer size by one thousand, million,

billion, or trillion, respectively.  The default is 5 Mbytes,

but will be increased if necessary to hold at least one chunk of

netCDF-4 chunked variables in the input file.  You may want to specify

a value larger than the default for copying large files over high

latency networks.  Using the '-w' option may provide better

performance, if the output fits in memory.

@par -e \e chunk_cache

@par

For netCDF-4 output, including netCDF-4 classic model, an integer or

floating-point number that specifies the size in bytes of chunk cache

for chunked variables.  This is not a property of the file, but merely

a performance tuning parameter for avoiding compressing or

decompressing the same data multiple times while copying and changing

chunk shapes.  A suffix of K, M, G, or T multiplies the chunk cache

size by one thousand, million, billion, or trillion, respectively.

The default is 4.194304 Mbytes (or whatever was specified for the

configure-time constant CHUNK_CACHE_SIZE when the netCDF library was

built).  Ideally, the \b nccopy utility should accept only one memory

buffer size and divide it optimally between a copy buffer and chunk

cache, but no general algorithm for computing the optimum chunk cache

size has been implemented yet. Using the '-w' option may provide

better performance, if the output fits in memory.

@par -h \e cache_elems

@par

For netCDF-4 output, including netCDF-4 classic model, specifies

number of elements that the chunk cache can hold. This is not a

property of the file, but merely a performance tuning parameter for

avoiding compressing or decompressing the same data multiple times

while copying and changing chunk shapes.  The default is 1009 (or

whatever was specified for the configure-time constant

CHUNK_CACHE_NELEMS when the netCDF library was built).  Ideally, the

\b nccopy utility should determine an optimum value for this parameter,

but no general algorithm for computing the optimum number of chunk

cache elements has been implemented yet.

@par -r

Read netCDF classic or 64-bit offset input file into a diskless netCDF

file in memory before copying.  Requires that input file be small

enough to fit into memory.  For \b nccopy, this doesn't seem to provide

any significant speedup, so may not be a useful option.

@section  EXAMPLES

@subsection simple_copy Simple Copy

Make  a copy  of  foo1.nc, a  netCDF  file of  any type,  to

foo2.nc, a netCDF file of the same type:

\code

nccopy foo1.nc foo2.nc

\endcode

Note that the above copy will not be as fast as use of cp or other

simple copy utility, because the file is copied using only the netCDF

API.  If the input file has extra bytes after the end of the netCDF

data, those will not be copied, because they are not accessible

through the netCDF interface.  If the original file was generated in

'No fill' mode so that fill values are not stored for padding for data

alignment, the output file may have different padding bytes.

@subsection uncompress Uncompress Data

Convert a netCDF-4 classic model file, compressed.nc, that uses

compression, to a netCDF-3 file classic.nc:

\code

nccopy -k classic compressed.nc classic.nc

\endcode

Note that '1' could be used instead of 'classic'.

@subsection remote_access Remote Access to Data Subset

Download the variable 'time_bnds' and its associated attributes from

an OPeNDAP server and copy the result to a netCDF file named 'tb.nc':

\code

nccopy 'http://test.opendap.org/opendap/data/nc/sst.mnmean.nc.gz?time_bnds' tb.nc

\endcode

Note that URLs that name specific variables as command-line arguments

should generally be quoted, to avoid the shell interpreting special

characters such as '?'.

@subsection compress Compress Data

Compress all the variables in the input file foo.nc, a netCDF file of

any type, to the output file bar.nc:

\code

nccopy -d1 foo.nc bar.nc

\endcode

If foo.nc was a classic or 64-bit offset netCDF file, bar.nc will be a

netCDF-4 classic model netCDF file, because the classic and 64-bit

offset format variants don't support compression.  If foo.nc was a

netCDF-4 file with some variables compressed using various deflation

levels, the output will also be a netCDF-4 file of the same type, but

all the variables, including any uncompressed variables in the input,

will now use deflation level 1.

@subsection rechunk Rechunk Data for Faster Access

Assume the input data includes gridded variables that use time, lat,

lon dimensions, with 1000 times by 1000 latitudes by 1000 longitudes,

and that the time dimension varies most slowly.  Also assume that

users want quick access to data at all times for a small set of

lat-lon points.  Accessing data for 1000 times would typically require

accessing 1000 disk blocks, which may be slow.

Reorganizing the data into chunks on disk that have all the time in

each chunk for a few lat and lon coordinates would greatly speed up

such access.  To chunk the data in the input file slow.nc, a netCDF

file of any type, to the output file fast.nc, you could use;

\code

nccopy -c time/1000,lat/40,lon/40 slow.nc fast.nc

\endcode

to specify data chunks of 1000 times, 40 latitudes, and 40 longitudes.

If you had enough memory to contain the output file, you could speed

up the rechunking operation significantly by creating the output in

memory before writing it to disk on close:

\code

nccopy -w -c time/1000,lat/40,lon/40 slow.nc fast.nc

\endcode

\page guide_ncgen3 ncgen3

The ncgen3 tool is the new name for the older, original ncgen utility.

The ncgen3 tool generates a netCDF file or a C or FORTRAN program that

creates a netCDF dataset. If no options are specified in invoking

ncgen3, the program merely checks the syntax of the CDL input,

producing error messages for any violations of CDL syntax.

The ncgen3 utility can only generate classic-model netCDF-4 files or

programs.

UNIX syntax for invoking ncgen3:

\code

ncgen3 [-b] [-o netcdf-file] [-c] [-f] [-v2|-v3] [-x] [input-file]

\endcode

where:

<pre>

-b

Create a (binary) netCDF file. If the '-o' option is absent, a default

file name will be constructed from the netCDF name (specified after

the netcdf keyword in the input) by appending the '.nc'

extension. Warning: if a file already exists with the specified name

it will be overwritten.

-o netcdf-file

Name for the netCDF file created. If this option is specified, it

implies the '-b' option. (This option is necessary because netCDF

files are direct-access files created with seek calls, and hence

cannot be written to standard output.)

-c

Generate C source code that will create a netCDF dataset matching the

netCDF specification. The C source code is written to standard

output. This is only useful for relatively small CDL files, since all

the data is included in variable initializations in the generated

program.

-f

Generate FORTRAN source code that will create a netCDF dataset

matching the netCDF specification. The FORTRAN source code is written

to standard output. This is only useful for relatively small CDL

files, since all the data is included in variable initializations in

the generated program.

-v2

The generated netCDF file or program will use the version of the

format with 64-bit offsets, to allow for the creation of very large

files. These files are not as portable as classic format netCDF files,

because they require version 3.6.0 or later of the netCDF library.

-v3

The generated netCDF file will be in netCDF-4/HDF5 format. These files

are not as portable as classic format netCDF files, because they

require version 4.0 or later of the netCDF library.

-x

Use “no fill” mode, omitting the initialization of variable values

with fill values. This can make the creation of large files much

faster, but it will also eliminate the possibility of detecting the

inadvertent reading of values that haven't been written.

</pre>

\page classic_format_spec The NetCDF Classic Format Specification

To present the format more formally, we use a BNF grammar notation. In

this notation:

- Non-terminals (entities defined by grammar rules) are in lower case.

- Terminals (atomic entities in terms of which the format

specification is written) are in upper case, and are specified

literally as US-ASCII characters within single-quote characters or are

described with text between angle brackets (‘\<’ and ‘\>’).

- Optional entities are enclosed between braces (‘[’ and ‘]’).

- A sequence of zero or more occurrences of an entity is denoted by

‘[entity ...]’.

- A vertical line character (‘|’) separates alternatives. Alternation

has lower precedence than concatenation.

- Comments follow ‘//’ characters.

- A single byte that is not a printable character is denoted using a

hexadecimal number with the notation ‘\\xDD’, where each D is a

hexadecimal digit.

- A literal single-quote character is denoted by ‘\'’, and a literal

back-slash character is denoted by ‘\\’.

Following the grammar, a few additional notes are included to specify

format characteristics that are impractical to capture in a BNF

grammar, and to note some special cases for implementers. Comments in

the grammar point to the notes and special cases, and help to clarify

the intent of elements of the format.

<h1>The Format in Detail</h1>

<pre>

netcdf_file  = header  data

header       = magic  numrecs  dim_list  gatt_list  var_list

magic        = 'C'  'D'  'F'  VERSION

VERSION      = \\x01 |                      // classic format

\\x02                        // 64-bit offset format

numrecs      = NON_NEG | STREAMING         // length of record dimension

dim_list     = ABSENT | NC_DIMENSION  nelems  [dim ...]

gatt_list    = att_list                    // global attributes

att_list     = ABSENT | NC_ATTRIBUTE  nelems  [attr ...]

var_list     = ABSENT | NC_VARIABLE   nelems  [var ...]

ABSENT       = ZERO  ZERO                  // Means list is not present

ZERO         = \\x00 \\x00 \\x00 \\x00         // 32-bit zero

NC_DIMENSION = \\x00 \\x00 \\x00 \\x0A         // tag for list of dimensions

NC_VARIABLE  = \\x00 \\x00 \\x00 \\x0B         // tag for list of variables

NC_ATTRIBUTE = \\x00 \\x00 \\x00 \\x0C         // tag for list of attributes

nelems       = NON_NEG       // number of elements in following sequence

dim          = name  dim_length

name         = nelems  namestring

// Names a dimension, variable, or attribute.

// Names should match the regular expression

// ([a-zA-Z0-9_]|{MUTF8})([^\\x00-\\x1F/\\x7F-\\xFF]|{MUTF8})*

// For other constraints, see "Note on names", below.

namestring   = ID1 [IDN ...] padding

ID1          = alphanumeric | '_'

IDN          = alphanumeric | special1 | special2

alphanumeric = lowercase | uppercase | numeric | MUTF8

lowercase    = 'a'|'b'|'c'|'d'|'e'|'f'|'g'|'h'|'i'|'j'|'k'|'l'|'m'|

'n'|'o'|'p'|'q'|'r'|'s'|'t'|'u'|'v'|'w'|'x'|'y'|'z'

uppercase    = 'A'|'B'|'C'|'D'|'E'|'F'|'G'|'H'|'I'|'J'|'K'|'L'|'M'|

'N'|'O'|'P'|'Q'|'R'|'S'|'T'|'U'|'V'|'W'|'X'|'Y'|'Z'

numeric      = '0'|'1'|'2'|'3'|'4'|'5'|'6'|'7'|'8'|'9'

// special1 chars have traditionally been

// permitted in netCDF names.

special1     = '_'|'.'|'@'|'+'|'-'

// special2 chars are recently permitted in

// names (and require escaping in CDL).

// Note: '/' is not permitted.

special2     = ' ' | '!' | '"' | '#'  | '$' | '%' | '&' | '\'' |

'(' | ')' | '*' | ','  | ':' | ';' | '<' | '='  |

'>' | '?' | '[' | '\\' | ']' | '^' | '`' | '{'  |

'|' | '}' | '~'

MUTF8        = <multibyte UTF-8 encoded, NFC-normalized Unicode character>

dim_length   = NON_NEG       // If zero, this is the record dimension.

// There can be at most one record dimension.

attr         = name  nc_type  nelems  [values ...]

nc_type      = NC_BYTE   |

NC_CHAR   |

NC_SHORT  |

NC_INT    |

NC_FLOAT  |

NC_DOUBLE

var          = name  nelems  [dimid ...]  vatt_list  nc_type  vsize  begin

// nelems is the dimensionality (rank) of the

// variable: 0 for scalar, 1 for vector, 2

// for matrix, ...

dimid        = NON_NEG       // Dimension ID (index into dim_list) for

// variable shape.  We say this is a "record

// variable" if and only if the first

// dimension is the record dimension.

vatt_list    = att_list      // Variable-specific attributes

vsize        = NON_NEG       // Variable size.  If not a record variable,

// the amount of space in bytes allocated to

// the variable's data.  If a record variable,

// the amount of space per record.  See "Note

// on vsize", below.

begin        = OFFSET        // Variable start location.  The offset in

// bytes (seek index) in the file of the

// beginning of data for this variable.

data         = non_recs  recs

non_recs     = [vardata ...] // The data for all non-record variables,

// stored contiguously for each variable, in

// the same order the variables occur in the

// header.

vardata      = [values ...]  // All data for a non-record variable, as a

// block of values of the same type as the

// variable, in row-major order (last

// dimension varying fastest).

recs         = [record ...]  // The data for all record variables are

// stored interleaved at the end of the

// file.

record       = [varslab ...] // Each record consists of the n-th slab

// from each record variable, for example

// x[n,...], y[n,...], z[n,...] where the

// first index is the record number, which

// is the unlimited dimension index.

varslab      = [values ...]  // One record of data for a variable, a

// block of values all of the same type as

// the variable in row-major order (last

// index varying fastest).

values       = bytes | chars | shorts | ints | floats | doubles

string       = nelems  [chars]

bytes        = [BYTE ...]  padding

chars        = [CHAR ...]  padding

shorts       = [SHORT ...]  padding

ints         = [INT ...]

floats       = [FLOAT ...]

doubles      = [DOUBLE ...]

padding      = <0, 1, 2, or 3 bytes to next 4-byte boundary>

// Header padding uses null (\\x00) bytes.  In

// data, padding uses variable's fill value.

// See "Note on padding", below, for a special

// case.

NON_NEG      = <non-negative INT>

STREAMING    = \\xFF \\xFF \\xFF \\xFF   // Indicates indeterminate record

// count, allows streaming data

OFFSET       = <non-negative INT> |  // For classic format or

<non-negative INT64>  // for 64-bit offset format

BYTE         = <8-bit byte>          // See "Note on byte data", below.

CHAR         = <8-bit byte>          // See "Note on char data", below.

SHORT        = <16-bit signed integer, Bigendian, two's complement>

INT          = <32-bit signed integer, Bigendian, two's complement>

INT64        = <64-bit signed integer, Bigendian, two's complement>

FLOAT        = <32-bit IEEE single-precision float, Bigendian>

DOUBLE       = <64-bit IEEE double-precision float, Bigendian>

// following type tags are 32-bit integers

NC_BYTE      = \\x00 \\x00 \\x00 \\x01       // 8-bit signed integers

NC_CHAR      = \\x00 \\x00 \\x00 \\x02       // text characters

NC_SHORT     = \\x00 \\x00 \\x00 \\x03       // 16-bit signed integers

NC_INT       = \\x00 \\x00 \\x00 \\x04       // 32-bit signed integers

NC_FLOAT     = \\x00 \\x00 \\x00 \\x05       // IEEE single precision floats

NC_DOUBLE    = \\x00 \\x00 \\x00 \\x06       // IEEE double precision floats

// Default fill values for each type, may be

// overridden by variable attribute named

// `_FillValue'. See "Note on fill values",

// below.

FILL_CHAR    = \\x00                      // null byte

FILL_BYTE    = \\x81                      // (signed char) -127

FILL_SHORT   = \\x80 \\x01                 // (short) -32767

FILL_INT     = \\x80 \\x00 \\x00 \\x01       // (int) -2147483647

FILL_FLOAT   = \\x7C \\xF0 \\x00 \\x00       // (float) 9.9692099683868690e+36

FILL_DOUBLE  = \\x47 \\x9E \\x00 \\x00 \\x00 \\x00 //(double)9.9692099683868690e+36

</pre>

Note on vsize: This number is the product of the dimension lengths

(omitting the record dimension) and the number of bytes per value

(determined from the type), increased to the next multiple of 4, for

each variable. If a record variable, this is the amount of space per

record (except that, for backward compatibility, it always includes

padding to the next multiple of 4 bytes, even in the exceptional case

noted below under “Note on padding”). The netCDF “record size” is

calculated as the sum of the vsize's of all the record variables.

The vsize field is actually redundant, because its value may be

computed from other information in the header. The 32-bit vsize field

is not large enough to contain the size of variables that require more

than 2^32 - 4 bytes, so 2^32 - 1 is used in the vsize field for such

variables.

Note on names: Earlier versions of the netCDF C-library reference

implementation enforced a more restricted set of characters in

creating new names, but permitted reading names containing arbitrary

bytes. This specification extends the permitted characters in names to

include multi-byte UTF-8 encoded Unicode and additional printing

characters from the US-ASCII alphabet. The first character of a name

must be alphanumeric, a multi-byte UTF-8 character, or '_' (reserved

for special names with meaning to implementations, such as the

“_FillValue” attribute). Subsequent characters may also include

printing special characters, except for '/' which is not allowed in

names. Names that have trailing space characters are also not

permitted.

Implementations of the netCDF classic and 64-bit offset format must

ensure that names are normalized according to Unicode NFC

normalization rules during encoding as UTF-8 for storing in the file

header. This is necessary to ensure that gratuitous differences in the

representation of Unicode names do not cause anomalies in comparing

files and querying data objects by name.

Note on streaming data: The largest possible record count, 2^32 - 1,

is reserved to indicate an indeterminate number of records. This means

that the number of records in the file must be determined by other

means, such as reading them or computing the current number of records

from the file length and other information in the header. It also

means that the numrecs field in the header will not be updated as

records are added to the file. [This feature is not yet implemented].

Note on padding: In the special case when there is only one record

variable and it is of type character, byte, or short, no padding is

used between record slabs, so records after the first record do not

necessarily start on four-byte boundaries. However, as noted above

under “Note on vsize”, the vsize field is computed to include padding

to the next multiple of 4 bytes. In this case, readers should ignore

vsize and assume no padding. Writers should store vsize as if padding

were included.

Note on byte data: It is possible to interpret byte data as either

signed (-128 to 127) or unsigned (0 to 255). When reading byte data

through an interface that converts it into another numeric type, the

default interpretation is signed. There are various attribute

conventions for specifying whether bytes represent signed or unsigned

data, but no standard convention has been established. The variable

attribute “_Unsigned” is reserved for this purpose in future

implementations.

Note on char data: Although the characters used in netCDF names must

be encoded as UTF-8, character data may use other encodings. The

variable attribute “_Encoding” is reserved for this purpose in future

implementations.

Note on fill values: Because data variables may be created before

their values are written, and because values need not be written

sequentially in a netCDF file, default “fill values” are defined for

each type, for initializing data values before they are explicitly

written. This makes it possible to detect reading values that were

never written. The variable attribute “_FillValue”, if present,

overrides the default fill value for a variable. If _FillValue is

defined then it should be scalar and of the same type as the variable.

Fill values are not required, however, because netCDF libraries have

traditionally supported a “no fill” mode when writing, omitting the

initialization of variable values with fill values. This makes the

creation of large files faster, but also eliminates the possibility of

detecting the inadvertent reading of values that haven't been written.

\page computing_offsets Notes on Computing File Offsets

The offset (position within the file) of a specified data value in a

classic format or 64-bit offset data file is completely determined by

the variable start location (the offset in the begin field), the

external type of the variable (the nc_type field), and the dimension

indices (one for each of the variable's dimensions) of the value

desired.

The external size in bytes of one data value for each possible netCDF

type, denoted extsize below, is:

- NC_BYTE 	1

- NC_CHAR 	1

- NC_SHORT 	2

- NC_INT 	4

- NC_FLOAT 	4

- NC_DOUBLE 	8

The record size, denoted by recsize below, is the sum of the vsize

fields of record variables (variables that use the unlimited

dimension), using the actual value determined by dimension sizes and

variable type in case the vsize field is too small for the variable

size.

To compute the offset of a value relative to the beginning of a

variable, it is helpful to precompute a “product vector” from the

dimension lengths. Form the products of the dimension lengths for the

variable from right to left, skipping the leftmost (record) dimension

for record variables, and storing the results as the product vector

for each variable.

For example:

\code

Non-record variable:

dimension lengths: [ 5 3 2 7] product vector: [210 42 14 7]

Record variable:

dimension lengths: [0 2 9 4] product vector: [0 72 36 4]

\endcode

At this point, the leftmost product, when rounded up to the next

multiple of 4, is the variable size, vsize, in the grammar above. For

example, in the non-record variable above, the value of the vsize

field is 212 (210 rounded up to a multiple of 4). For the record

variable, the value of vsize is just 72, since this is already a

multiple of 4.

Let coord be the array of coordinates (dimension indices, zero-based)

of the desired data value. Then the offset of the value from the

beginning of the file is just the file offset of the first data value

of the desired variable (its begin field) added to the inner product

of the coord and product vectors times the size, in bytes, of each

datum for the variable. Finally, if the variable is a record variable,

the product of the record number, 'coord[0]', and the record size,

recsize, is added to yield the final offset value.

A special case: Where there is exactly one record variable, we drop

the requirement that each record be four-byte aligned, so in this case

there is no record padding.

\page offset_examples Examples

By using the grammar above, we can derive the smallest valid netCDF

file, having no dimensions, no variables, no attributes, and hence, no

data. A CDL representation of the empty netCDF file is

\code

netcdf empty { }

\endcode

This empty netCDF file has 32 bytes. It begins with the four-byte

“magic number” that identifies it as a netCDF version 1 file: ‘C’,

‘D’, ‘F’, ‘\\x01’. Following are seven 32-bit integer zeros

representing the number of records, an empty list of dimensions, an

empty list of global attributes, and an empty list of variables.

Below is an (edited) dump of the file produced using the Unix command

\code

od -xcs empty.nc

\endcode

Each 16-byte portion of the file is displayed with 4 lines. The first

line displays the bytes in hexadecimal. The second line displays the

bytes as characters. The third line displays each group of two bytes

interpreted as a signed 16-bit integer. The fourth line (added by

human) presents the interpretation of the bytes in terms of netCDF

components and values.

\code

4344    4601    0000    0000    0000    0000    0000    0000

C   D   F 001  \0  \0  \0  \0  \0  \0  \0  \0  \0  \0  \0  \0

17220   17921   00000   00000   00000   00000   00000   00000

[magic number ] [  0 records  ] [  0 dimensions   (ABSENT)    ]

0000    0000    0000    0000    0000    0000    0000    0000

\0  \0  \0  \0  \0  \0  \0  \0  \0  \0  \0  \0  \0  \0  \0  \0

00000   00000   00000   00000   00000   00000   00000   00000

[  0 global atts  (ABSENT)    ] [  0 variables    (ABSENT)    ]

\endcode

As a less trivial example, consider the CDL

\code

netcdf tiny {

dimensions:

dim = 5;

variables:

short vx(dim);

data:

vx = 3, 1, 4, 1, 5 ;

}

\endcode

which corresponds to a 92-byte netCDF file. The following is an edited dump of this file:

\code

4344    4601    0000    0000    0000    000a    0000    0001

C   D   F 001  \0  \0  \0  \0  \0  \0  \0  \n  \0  \0  \0 001

17220   17921   00000   00000   00000   00010   00000   00001

[magic number ] [  0 records  ] [NC_DIMENSION ] [ 1 dimension ]

0000    0003    6469    6d00    0000    0005    0000    0000

\0  \0  \0 003   d   i   m  \0  \0  \0  \0 005  \0  \0  \0  \0

00000   00003   25705   27904   00000   00005   00000   00000

[  3 char name = "dim"        ] [ size = 5    ] [ 0 global atts

0000    0000    0000    000b    0000    0001    0000    0002

\0  \0  \0  \0  \0  \0  \0 013  \0  \0  \0 001  \0  \0  \0 002

00000   00000   00000   00011   00000   00001   00000   00002

(ABSENT)     ] [NC_VARIABLE  ] [ 1 variable  ] [ 2 char name =

7678    0000    0000    0001    0000    0000    0000    0000

v   x  \0  \0  \0  \0  \0 001  \0  \0  \0  \0  \0  \0  \0  \0

30328   00000   00000   00001   00000   00000   00000   00000

"vx"         ] [1 dimension  ] [ with ID 0   ] [ 0 attributes

0000    0000    0000    0003    0000    000c    0000    0050

\0  \0  \0  \0  \0  \0  \0 003  \0  \0  \0  \f  \0  \0  \0   P

00000   00000   00000   00003   00000   00012   00000   00080

(ABSENT)     ] [type NC_SHORT] [size 12 bytes] [offset:    80]

0003    0001    0004    0001    0005    8001

\0 003  \0 001  \0 004  \0 001  \0 005 200 001

00003   00001   00004   00001   00005  -32767

[    3] [    1] [    4] [    1] [    5] [fill ]

\endcode

\page offset_format_spec The 64-bit Offset Format

The netCDF 64-bit offset format differs from the classic format only

in the VERSION byte, ‘\\x02’ instead of ‘\\x01’, and the OFFSET entity,

a 64-bit instead of a 32-bit offset from the beginning of the

file. This small format change permits much larger files, but there

are still some practical size restrictions. Each fixed-size variable

and the data for one record's worth of each record variable are still

limited in size to a little less that 4 GiB. The rationale for this

limitation is to permit aggregate access to all the data in a netCDF

variable (or a record's worth of data) on 32-bit platforms.

\page netcdf_4_spec The NetCDF-4 Format

The netCDF-4 format implements and expands the netCDF-3 data model by

using an enhanced version of HDF5 as the storage layer. Use is made of

features that are only available in HDF5 version 1.8 and later.

Using HDF5 as the underlying storage layer, netCDF-4 files remove many

of the restrictions for classic and 64-bit offset files. The richer

enhanced model supports user-defined types and data structures,

hierarchical scoping of names using groups, additional primitive types

including strings, larger variable sizes, and multiple unlimited

dimensions. The underlying HDF5 storage layer also supports

per-variable compression, multidimensional tiling, and efficient

dynamic schema changes, so that data need not be copied when adding

new variables to the file schema.

Creating a netCDF-4/HDF5 file with netCDF-4 results in an HDF5

file. The features of netCDF-4 are a subset of the features of HDF5,

so the resulting file can be used by any existing HDF5 application.

Although every file in netCDF-4 format is an HDF5 file, there are HDF5

files that are not netCDF-4 format files, because the netCDF-4 format

intentionally uses a limited subset of the HDF5 data model and file

format features. Some HDF5 features not supported in the netCDF

enhanced model and netCDF-4 format include non-hierarchical group

structures, HDF5 reference types, multiple links to a data object,

user-defined atomic data types, stored property lists, more permissive

rules for data object names, the HDF5 date/time type, and attributes

associated with user-defined types.

A complete specification of HDF5 files is beyond the scope of this

document. For more information about HDF5, see the HDF5 web site:

http://hdf.ncsa.uiuc.edu/HDF5/.

The specification that follows is sufficient to allow HDF5 users to

create files that will be accessable from netCDF-4.

\section creation_order Creation Order

The netCDF API maintains the creation order of objects that are

created in the file. The same is not true in HDF5, which maintains the

objects in alphabetical order. Starting in version 1.8 of HDF5, the

ability to maintain creation order was added. This must be explicitly

turned on in the HDF5 data file in several ways.

Each group must have link and attribute creation order set. The

following code (from libsrc4/nc4hdf.c) shows how the netCDF-4 library

sets these when creating a group.

\code

/* Create group, with link_creation_order set in the group

* creation property list. */

if ((gcpl_id = H5Pcreate(H5P_GROUP_CREATE)) < 0)

return NC_EHDFERR;

if (H5Pset_link_creation_order(gcpl_id, H5P_CRT_ORDER_TRACKED|H5P_CRT_ORDER_INDEXED) < 0)

BAIL(NC_EHDFERR);

if (H5Pset_attr_creation_order(gcpl_id, H5P_CRT_ORDER_TRACKED|H5P_CRT_ORDER_INDEXED) < 0)

BAIL(NC_EHDFERR);

if ((grp->hdf_grpid = H5Gcreate2(grp->parent->hdf_grpid, grp->name,

H5P_DEFAULT, gcpl_id, H5P_DEFAULT)) < 0)

BAIL(NC_EHDFERR);

if (H5Pclose(gcpl_id) < 0)

BAIL(NC_EHDFERR);

\endcode

Each dataset in the HDF5 file must be created with a property list for

which the attribute creation order has been set to creation

ordering. The H5Pset_attr_creation_order funtion is used to set the

creation ordering of attributes of a variable.

The following example code (from libsrc4/nc4hdf.c) shows how the

creation ordering is turned on by the netCDF library.

\code

/* Turn on creation order tracking. */

if (H5Pset_attr_creation_order(plistid, H5P_CRT_ORDER_TRACKED|

H5P_CRT_ORDER_INDEXED) < 0)

BAIL(NC_EHDFERR);

\endcode

\section groups_spec Groups

NetCDF-4 groups are the same as HDF5 groups, but groups in a netCDF-4

file must be strictly hierarchical. In general, HDF5 permits

non-hierarchical structuring of groups (for example, a group that is

its own grandparent). These non-hierarchical relationships are not

allowed in netCDF-4 files.

In the netCDF API, the global attribute becomes a group-level

attribute. That is, each group may have its own global attributes.

The root group of a file is named “/” in the netCDF API, where names

of groups are used. It should be noted that the netCDF API (like the

HDF5 API) makes little use of names, and refers to entities by number.

\section dims_spec Dimensions with HDF5 Dimension Scales

Until version 1.8, HDF5 did not have any capability to represent

shared dimensions. With the 1.8 release, HDF5 introduced the dimension

scale feature to allow shared dimensions in HDF5 files.

The dimension scale is unfortunately not exactly equivilent to the

netCDF shared dimension, and this leads to a number of compromises in

the design of netCDF-4.

A netCDF shared dimension consists solely of a length and a name. An

HDF5 dimension scale also includes values for each point along the

dimension, information that is (optionally) included in a netCDF

coordinate variable.

To handle the case of a netCDF dimension without a coordinate

variable, netCDF-4 creates dimension scales of type char, and leaves

the contents of the dimension scale empty. Only the name and length of

the scale are significant. To distinguish this case, netCDF-4 takes

advantage of the NAME attribute of the dimension scale. (Not to be

confused with the name of the scale itself.) In the case of dimensions

without coordinate data, the HDF5 dimension scale NAME attribute is

set to the string: "This is a netCDF dimension but not a netCDF

variable."

In the case where a coordinate variable is defined for a dimension,

the HDF5 dimscale matches the type of the netCDF coordinate variable,

and contains the coordinate data.

A further difficulty arrises when an n-dimensional coordinate variable

is defined, where n is greater than one. NetCDF allows such coordinate

variables, but the HDF5 model does not allow dimension scales to be

attached to other dimension scales, making it impossible to completely

represent the multi-dimensional coordinate variables of the netCDF

model.

To capture this information, multidimensional coordinate variables

have an attribute named _Netcdf4Coordinates. The attribute is an array

of H5T_NATIVE_INT, with the netCDF dimension IDs of each of its

dimensions.

The _Netcdf4Coordinates attribute is otherwise hidden by the netCDF

API. It does not appear as one of the attributes for the netCDF

variable involved, except through the HDF5 API.

\section dim_spec2 Dimensions without HDF5 Dimension Scales

Starting with the netCDF-4.1 release, netCDF can read HDF5 files which

do not use dimension scales. In this case the netCDF library assigns

dimensions to the HDF5 dataset as needed, based on the length of the

dimension.

When an HDF5 file is opened, each dataset is examined in turn. The

lengths of all the dimensions involved in the shape of the dataset are

determined. Each new (i.e. previously unencountered) length results in

the creation of a phony dimension in the netCDF API.

This will not accurately detect a shared, unlimited dimension in the

HDF5 file, if different datasets have different lengths along this

dimension (possible in HDF5, but not in netCDF).

Note that this is a read-only capability for the netCDF library. When

the netCDF library writes HDF5 files, they always use a dimension

scale for every dimension.

Datasets must have either dimension scales for every dimension, or no

dimension scales at all. Partial dimension scales are not, at this

time, understood by the netCDF library.

\section dim_spec3 Dimension and Coordinate Variable Ordering

In order to preserve creation order, the netCDF-4 library writes

variables in their creation order. Since some variables are also

dimension scales, their order reflects both the order of the

dimensions and the order of the coordinate variables.

However, these may be different. Consider the following code:

\code

/* Create a test file. */

if (nc_create(FILE_NAME, NC_CLASSIC_MODEL|NC_NETCDF4, &ncid)) ERR;

/* Define dimensions in order. */

if (nc_def_dim(ncid, DIM0, NC_UNLIMITED, &dimids[0])) ERR;

if (nc_def_dim(ncid, DIM1, 4, &dimids[1])) ERR;

/* Define coordinate variables in a different order. */

if (nc_def_var(ncid, DIM1, NC_DOUBLE, 1, &dimids[1], &varid[1])) ERR;

if (nc_def_var(ncid, DIM0, NC_DOUBLE, 1, &dimids[0], &varid[0])) ERR;

\endcode

In this case the order of the coordinate variables will be different

from the order of the dimensions.

In practice, this should make little difference in user code, but if

the user is writing code that depends on the ordering of dimensions,

the netCDF library was updated in version 4.1 to detect this

condition, and add the attribute _Netcdf4Dimid to the dimension scales

in the HDF5 file. This attribute holds a scalar H5T_NATIVE_INT which

is the (zero-based) dimension ID for this dimension.

If this attribute is present on any dimension scale, it must be

present on all dimension scales in the file.

\section vars_spec Variables

Variables in netCDF-4/HDF5 files exactly correspond to HDF5

datasets. The data types match naturally between netCDF and HDF5.

In netCDF classic format, the problem of endianness is solved by

writing all data in big-endian order. The HDF5 library allows data to

be written as either big or little endian, and automatically reorders

the data when it is read, if necessary.

By default, netCDF uses the native types on the machine which writes

the data. Users may change the endianness of a variable (before any

data are written). In that case the specified endian type will be used

in HDF5 (for example, a H5T_STD_I16LE will be used for NC_SHORT, if

little-endian has been specified for that variable.)

- NC_BYTE = H5T_NATIVE_SCHAR

- NC_UBYTE = H5T_NATIVE_SCHAR

- NC_CHAR = H5T_C_S1

- NC_STRING = variable length array of H5T_C_S1

- NC_SHORT = H5T_NATIVE_SHORT

- NC_USHORT = H5T_NATIVE_USHORT

- NC_INT = H5T_NATIVE_INT

- NC_UINT = H5T_NATIVE_UINT

- NC_INT64 = H5T_NATIVE_LLONG

- NC_UINT64 = H5T_NATIVE_ULLONG

- NC_FLOAT = H5T_NATIVE_FLOAT

- NC_DOUBLE = H5T_NATIVE_DOUBLE

The NC_CHAR type represents a single character, and the NC_STRING an

array of characters. This can be confusing because a one-dimensional

array of NC_CHAR is used to represent a string (i.e. a scalar

NC_STRING).

An odd case may arise in which the user defines a variable with the

same name as a dimension, but which is not intended to be the

coordinate variable for that dimension. In this case the string

"_nc4_non_coord_" is pre-pended to the name of the HDF5 dataset, and

stripped from the name for the netCDF API.

\section atts_spec Attributes

Attributes in HDF5 and netCDF-4 correspond very closely. Each

attribute in an HDF5 file is represented as an attribute in the

netCDF-4 file, with the exception of the attributes below, which are

ignored by the netCDF-4 API.

- _Netcdf4Coordinates An integer array containing the dimension IDs of

a variable which is a multi-dimensional coordinate variable.

- _nc3_strict When this (scalar, H5T_NATIVE_INT) attribute exists in

the root group of the HDF5 file, the netCDF API will enforce the

netCDF classic model on the data file.

- REFERENCE_LIST This attribute is created and maintained by the HDF5

dimension scale API.

- CLASS This attribute is created and maintained by the HDF5 dimension

scale API.

- DIMENSION_LIST This attribute is created and maintained by the HDF5

dimension scale API.

- NAME This attribute is created and maintained by the HDF5 dimension

scale API.

- _Netcdf4Dimid Holds a scalar H5T_NATIVE_INT that is the (zero-based)

dimension ID for this dimension, needed when dimensions and

coordinate variables are defined in different orders.

\section user_defined_spec User-Defined Data Types

Each user-defined data type in an HDF5 file exactly corresponds to a

user-defined data type in the netCDF-4 file. Only base data types

which correspond to netCDF-4 data types may be used. (For example, no

HDF5 reference data types may be used.)

\section compression_spec Compression

The HDF5 library provides data compression using the zlib library and

the szlib library. NetCDF-4 only allows users to create data with the

zlib library (due to licensing restrictions on the szlib

library). Since HDF5 supports the transparent reading of the data with

either compression filter, the netCDF-4 library can read data

compressed with szlib (if the underlying HDF5 library is built to

support szlib), but has no way to write data with szlib compression.

With zlib compression (a.k.a. deflation) the user may set a deflation

factor from 0 to 9. In our measurements the zero deflation level does

not compress the data, but does incur the performance penalty of

compressing the data. The netCDF API does not allow the user to write

a variable with zlib deflation of 0 - when asked to do so, it turns

off deflation for the variable instead. NetCDF can read an HDF5 file

with deflation of zero, and correctly report that to the user.

\page netcdf_4_classic_spec The NetCDF-4 Classic Model Format

Every classic and 64-bit offset file can be represented as a netCDF-4

file, with no loss of information. There are some significant benefits

to using the simpler netCDF classic model with the netCDF-4 file

format. For example, software that writes or reads classic model data

can write or read netCDF-4 classic model format data by

recompiling/relinking to a netCDF-4 API library, with no or only

trivial changes needed to the program source code. The netCDF-4

classic model format supports this usage by enforcing rules on what

functions may be called to store data in the file, to make sure its

data can be read by older netCDF applications (when relinked to a

netCDF-4 library).

Writing data in this format prevents use of enhanced model features

such as groups, added primitive types not available in the classic

model, and user-defined types. However performance features of the

netCDF-4 formats that do not require additional features of the

enhanced model, such as per-variable compression and chunking,

efficient dynamic schema changes, and larger variable size limits,

offer potentially significant performance improvements to readers of

data stored in this format, without requiring program changes.

When a file is created via the netCDF API with a CLASSIC_MODEL mode

flag, the library creates an attribute (_nc3_strict) in the root

group. This attribute is hidden by the netCDF API, but is read when

the file is later opened, and used to ensure that no enhanced model

features are written to the file.

\page hdf4_sd_format HDF4 SD Format

Starting with version 4.1, the netCDF libraries can read HDF4 SD

(Scientific Dataset) files. Access is limited to those HDF4 files

created with the Scientific Dataset API. Access is read-only.

Dataset types are translated between HDF4 and netCDF in a

straighforward manner.

- DFNT_CHAR = NC_CHAR

- DFNT_UCHAR, DFNT_UINT8 = NC_UBYTE

- DFNT_INT8 = NC_BYTE

- DFNT_INT16 = NC_SHORT

- DFNT_UINT16 = NC_USHORT

- DFNT_INT32 = NC_INT

- DFNT_UINT32 = NC_UINT

- DFNT_FLOAT32 = NC_FLOAT

- DFNT_FLOAT64 = NC_DOUBLE

\htmlonly

\page htmlonly_page html only page

LA LA LA!

\endhtmlonly

*/