General Conventions approved by NUWG

Variables

---------

The variable names chosen for netCDF variables will be up the supplier of the

data set. The agreement by this group is that all code written to read

netCDF data will NOT be variable name dependent. Code will be written

generically using the netCDF inquire functions or will employ some sort of

table look-up algorithm.

Attributes

----------

The group has agreed to attach "long_name" attributes to all variables as

a means of self-documenting files. When a user performs an ncdump, "long_name"

attributes should help that user understand what is contained in the file.

Any units used must be compatible with the UDUNITS library and consistent for

a given variable (For example, a data supplier chooses to store SAO temperatures

as degrees C. The supplier will convert all U.S. temperatures reported in

degrees F to degrees C).  The "units" attribute is required for all measured

or observed variables on which programs might perform numerical computations.

Time Representation

-------------------

The NUWG agreed that all time fields will be represented such to be compatible

with the UDUNITS library. We agreed that time should be represented as a

double such as:

double time

:units = 	<user defined, compatible with UDUNITS>

:long_name =	<user description>

:_FillValue = 	<user defined>

Defining time as a double gives the most flexibility to the widest range of

data sets.

Station Identification

----------------------

To identify fixed location stations, we agreed that the numeric WMO id would

be the identifier of choice. As mentioned above, data suppliers could add an

internal id in addition to the WMO id. To fully identify stations, a station

name, the latitude, longitude and elevation of the station will be required.

General Comments

----------------

In general, the netCDF definitions for a data set that we create are considered

a minimum set of variables for data exchange. The data provider is free to

add additional fields as needed for quality control, further identification,

or other internal uses.

-------------------------------------------------------------------------------

by Susan Jesuroga		jesuroga@comet.ucar.edu

for NUWG			nuwg@comet.ucar.edu

Updated October 5, 1993